Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRIC CIRCUIT ANALYSIS					      	     Time : 3 hours

Subject Code:	12EC202						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Kirchoff’s current law.

2.	A 10Ω resistor is connected across a 12 V battery. How much current flows through the resistor?

3.	What is meant by resonance frequencies?

4.	Mention the two ways of interconnecting the three phase sources in a three phase balanced circuit.

5.	Define mesh.

6.	Two inductively coupled coils have self inductances L1 =50mH and L2 = 200mH. If the coefficient of coupling is 0.5, find the value of mutual inductance between the coils?

7.	What is the condition for maximum power transfer theorem?

8.	Norton’s theorem is dual of ______________ theorem.

9.	What is meant by transient response?

10.	The time constant of a series RC circuit is (a) RC   (b) C/R   (c) R/C.

PART – B  (5 x 3 = 15 MARKS)

11.	Use voltage division rule to determine Vx in the circuit.

12.	For the circuit given, determine the value of Quality factor (Q) at resonance and Bandwidth of the circuit.

13.	What is the maximum possible mutual inductance for L1=50mH and L2= 200mH and also find the effective inductance of the combination for the circuit given.

[P.T.O]

14.	In the Circuit shown, determine the value of load resistance when the load resistance draws 	maximum power. Also find the value of maximum power.

15.	A series RL circuit with R=30Ω and L=15H has a constant voltage V=60V applied at t=0 as shown in the circuit. Determine the current I, the voltage across resistor and the voltage across the inductor.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Determine Vx in the circuit.													  (12)

b.	Write an expression for the current through the 3Ω resistor in the circuit given.		    (3)

(OR)

17.	a.	Use the technique of star-delta conversion to find the equivalent resistance of the given 	circuit. Each resistor R is 10Ω.													  (12)

b.	For the Circuit given, compute the current Ix through 47KΩ resistor after performing a source transformation on the voltage source.										    (3)

18.	a.	For the circuit shown, determine the impedance at resonant frequency, 10Hz above 	resonant frequency and 10Hz below resonant frequency.							  (12)

b.	Find the resonant frequency of the series RLC Circuit with R= 10Ω, L=0.5mH and C=10µF. 																	    (3)

(OR)

19.	a.	An unbalanced four-wire, star connected load has a balanced voltage of 400 V and the 	loads are Z1= (4+j8)Ω; Z2=(3+j4)Ω; Z3= (15+j20)Ω. Calculate (i) Line currents (ii) current 	in neutral wire (iii) Total power.												  (10)

b.	Derive an expression for power factor by two wattmeter method.					    (5)

20.	Use mesh analysis to determine the three mesh currents in the circuit.

(OR)

21.	Use Nodal Analysis to determine the current in each branch for the given network.

[P.T.O]

22.	Compute the current in 23Ω resistor by applying superposition principle.

(OR)

23.	Apply Thevenin theorem to determine the current through 5Ω resistor in the circuit shown.

24.	a.	A series RC circuit consists of resistor of 100Ω and capacitor of 0.01F as shown in the 	circuit. A constant voltage of 20V is applied to the circuit at t=0. Obtain the current 	equation. Determine the voltages across the resistor and the capacitor.				    (7)

b.	The circuit shown consists of resistance, inductance and capacitance in series with a 100V 	constant source when the switch is closed at t=0. Find the current transient.			    (8)

(OR)

25.	Derive an expression for the current of the given RC circuit.