Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	SOLID STATE CIRCUITS							      	     Time : 3 hours

Subject Code:	12EC207						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define peak inverse voltage.

2.	Which active device is basically employed in shunt voltage regulator?

3.	What is meant by transistor biasing?

4.	Define stability factor.

5.	RC coupling is used for __________ amplification.

a)voltage 	b)current	c)power

6.	The maximum collector efficiency of class B amplifier is __________.

7.	Give one application of positive feedback amplifier.

8.	Mention one advantage of negative feedback amplifier.

9.	Write one application of single tuned amplifiers.

10.	Write the equation for frequency of oscillations in Colpitts oscillators.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare the full wave rectifier with half wave rectifier circuit.

12.	Consider the fixed bias BJT circuit having VCC =12V, IB =40 µA, VCE = 6V, β=80, VBE  =0.7 V.  	Determine a) Collector  current  b) Collector Resistance

13.	Explain the frequency response of RC coupled amplifier with a neat diagram.

14.	Draw the voltage feedback circuit and mention its significance.

15.	What is an Oscillator? State Barkhausan criterion.

PART – C  (5 x 15 = 75 MARKS)

16.	With a circuit diagram and necessary waveforms describe the operation of a full wave rectifier and derive the expression for ripple factor and rectification efficiency.

(OR)

17.	Draw a neat circuit diagram of short circuit or over load protection and explain its working.

18.	Draw the fixed biasing circuit of BJT and derive its stability factor.

(OR)

19.	Determine the following for the FET self-bias network VGSQ, IDQ, VDS, VS, VG, VD with the following specifications. [VDD=20 V, RD=3.3 kΩ, RS=1 kΩ, RG=1 MΩ, IDSS=8 mA, VP = –6 V]

20.	Draw the circuit diagram of RC coupled amplifier and explain its working with frequency   response.

(OR)

21.	Explain the operation of complementary symmetry Push pull amplifier with neat circuit diagram and obtain its collector efficiency.

[P.T.O.]

22.	Draw the circuit diagram of voltage series feedback amplifier and derive its input impedance, output impedance and voltage gain.

(OR)

23.	Draw the circuit diagram of Differential amplifier using BJT and explain its operation.

24.	Explain the operation of RC phase shift oscillator with neat diagram and also mention its advantages and disadvantages.

(OR)

25.	With a neat sketch, describe the functioning of single tuned amplifier.