Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	LINEAR INTEGRATED CIRCUITS AND APPLICATIONS													      	     								      Time : 3 hours

Subject Code:	12EC208								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define Slew rate.

2. 	Name any one method used to provide external frequency compensation.

3. 	In RC phase shift oscillator, the frequency of oscillation f0 is _________.

4. 	What are the applications of comparator using op-amp?

5.  	Draw the frequency of high pass filter.

6. 	Mention the two types of band pass filter.

7. 	Define capture range.

8. 	List the applications of PLL.

9. 	What is meant by etching?

10. 	Mention the advantages of ICs.

PART – B  (5 x 3 = 15 MARKS)

11. 	List out the ideal characteristics of op-amp.

12. 	Describe the operation of series op-amp regulator.

13. 	Determine the order of a low-pass Butterworth filter that is to provide 40dB attenuation at

ω/ωh=2.

14. 	Explain how PLL can be used as a AM demodulator.

15. 	Describe the fabrication process of diffused resistor.

PART – C  (5 x 15 = 75 MARKS)

16. 	Derive the gain expression for inverting and non-inverting amplifier using op-amp with neat circuit diagram.

(OR)

17.  	With neat sketch explain the operation of Log amplifier and determine its output voltage expression.

18. 	Draw the circuit diagram of an astable multivibrator using op-amp and derive an expression for its frequency of oscillation.

(OR)

19. 	With a neat diagram, explain about IC723 general purpose regulator.

20. 	Determine the transfer function of first order low pass filter and draw its frequency response.

(OR)

21. 	Draw the functional diagram of IC 555 timer and explain its operation.

22. 	Explain the working principle of PLL with neat block diagram.

(OR)

23. 	With the help of a neat block diagram, explain the operation of Successive approximation ADC.

24. 	Describe basic processes of fabrication of Integrated Circuits using silicon planar technology.

(OR)

25. 	Discuss in detail about the fabrication steps of CMOS.