Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	TRANSMISSION LINES AND WAVE GUIDES		      	     Time : 3 hours

Subject Code:	12EC211								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw the equivalent circuit model of a transmission line.

2.	Write the relationship between neper and decibel.

3.	VSWR of a perfectly matched transmission line is _________.

4.	On a transmission line with standing waves, give the distance between a voltage maxima and 	adjacent current maxima.

5.	State the condition for impedance matching in a transmission line.

6.	Write any two applications of smith chart.

7.	The ratio of the speed at which a wavefront of an acoustic signal passes through the medium, to 	the speed of light in a vacuum is called _________.

8.	Give the cut-off frequency when the wave is propagated by between two parallel plates for 	TEM mode.

9.	Indicate the electric and magnetic field distributions of a rectangular waveguide carrying TE10

wave.

10.	What is a cavity resonator?

PART – B  (5 x 3 = 15 MARKS)

11.	Write a note on Waveform Distortion.

12.	A transmission line with characteristic impedance of 500Ω is terminated by a purely resistive 	load. It is found by measurement that the minimum value of voltage upon it is 5µV and 	maximum voltage is 7.55µV. Calculate the value of load resistance.

13.	Derive the input impedance of a half wave transmission line.

14.	Write the Maxwell’s equation for free space (Point form only).

15.	Why TEM mode does not exist in a rectangular waveguide?

PART – C  (5 x 15 = 75 MARKS)

16.	With necessary equations, discuss about the types of transmission lines and their losses.

(OR)

17.	Derive Campbell’s equation for an equivalent T section of a transmission line consisting 	partially of lumped and partially of distributed elements.

18.	18. 	A line with zero dissipation has R = 0.006 Ω/m, L = 2.5μH/m, C= 4.45pF/m. If the line is 	operated at 10MHz find

a.	R0 			b.	ß 		c.	V 		d.	λ

(OR)

19.	With necessary equations, describe in detail, standing wave patterns and standing wave ratio on a line.

[P.T.O]

20.	Discuss the following:

a.	Half Wave line																    (7)

b.	Single stub impedance matching												    (8)

(OR)

21.	A lossless transmission line with a characteristic impedance of 50Ω is terminated in a load     (50 + j50) Ω. Using the Smith chart find the following:

a.	VSWR.

b.	The magnitude and the angle of reflection coefficient at the load.

c.	Load admittance.

d.	Reflection coefficient at a distance of λ/4 from the load.

e.	The position of the first voltage minimum from the load.

22.	A pair of perfectly conducting planes is separated by 8 cm in air. For frequency of 5000MHz

with TM1 mode excited, find the following

a.	Cut- off frequency. 															    (2)

b.	Characteristic impedance. 														    (2)

f.	Attenuation constant for f=0.95fc. 												    (3)

d.	Phase shift. 																	    (2)

e.	Phase velocity and group velocity. 												    (4)

f.	Wavelength measured along the guided walls. 									    (2)

(OR)

23.	Write short notes on

a.	Attenuation in parallel plane waves.											    (7)

b.	Wave Impedance 															    (8)

24.	a.	With neat diagrams, explain the excitation of modes in Rectangular waveguides.		  (10)

b.	Derive the wave impedance for TE waves in a Rectangular waveguides.                          (5) (OR)

25.	Derive the attenuation constant for TE10 mode in a rectangular waveguide.