Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	PULSE AND WAVE SHAPING CIRCUITS  			      	     Time : 3 hours

Subject Code:	12EC212								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by linear waveshaping?

2.	List any two non-linear waveshaping circuits.

3.	Identify the multivibrator that can be used as a memory element as well as counting element in digital circuits.

4.	Mention the application of the Schmitt trigger circuit.

5.	How do you justify that a one-shot Multivibrator is also called a gating circuit?

6.	The Astable Multivibrator has two Quasi-stable state. [True/False]

7.	What is meant by Restoration Time?

8.	Mention any one method of generating a time base waveform.

9.	The blocking oscillator as a low impedance switch used to discharge a capacitor quickly. [True/False]

10.	Why sampling gate is also referred to as a linear gate?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate Clippers and Clampers.

12.	Mention the methods for triggering bistable multivibrator and distinguish between them.

13.	A collector coupled monostable multivibrator using two 38N3903 NPN silicon annular transistors has the following parameters: VCC = 15V, VBB = 5V, RC = 2.3kΩ, R1 = R2 =R = 25kΩ, hfe =30, rbb’ = 200Ω and C=900pF. Determine the gate width of the output waveform. The reverse saturation current ICBO may be neglected.

14.	Obtain expressions for slope error and sweep speed in the miller sweep voltage waveform generator.

15.	Mention some of the applications of blocking oscillator.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A symmetrical square wave of peak-to-peak amplitude V and frequency f is applied to a 	high pass RC circuit. Show that the percentage tilt is given by

P=T2RC×100%.															  (12)

b.	Sketch the response of an RC low pass circuit for (Assume that RC >>T)

(i) Square Input	    (ii) Ramp Input	  										    (3)

(OR)

17.	a.	With Neat Circuit Diagram and output Waveforms, explain the working principle of 	positive and negative clampers.												    (8)

b.	State Clamping circuit theorem and mathematically prove it.						    (7)

18.	With neat circuit diagram, explain the operation of Fixed bias bistable multivibrator and analyze the circuit to find its stable state currents and voltages.

[P.T.O]

(OR)

19.	Make a quantitative analysis of the transistor Schmitt trigger to obtain an expression for the critical voltages VUTP and VLTP with neat circuit diagram.

20.	Explain the operation of collector coupled astable multivibrator with neat circuit diagram and Derive an expression for Time period T.

(OR)

21.	Consider an emitter coupled monostable multivibrator using NPN transistors. The circuit and device parameters are VCC = 18V, R = 100kΩ, RC1 = 6kΩ, RC2 = 5kΩ, V= 5V, RE = 4kΩ, hFE = 50, rbb’ =200Ω. Assume germanium transistor for which VBE(sat)  = 0.3V, VCE(sat)  = 0.1V and cut-in voltage = 0.1V Calculate the voltages at the base B2, collector  C1, C2 and common emitter E.

22.	Prove that the relationship between displacement error (ed), transmission error (et) and slope error (es) is

ed= es / 8 = et /4

(OR)

23.	Describe the working principle of transistor current sweep circuit with neat diagram.

24.	Make use of neat sketches of circuits, analyse the triggered transistor monostable blocking oscillator with emitter timing to obtain an expression for the pulse width tp.

(OR)

25.	With the help of equivalent circuit of Bidirectional diode sampling gate, derive an expression for Gain (A), control voltage (VCmin) and control voltage (VNmin).