Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	COMMUNICATION THEORY						      	     Time : 3 hours

Subject Code:	12EC213						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the advantages of DSB-SC over Amplitude modulation?

2.	A transmitter supplies an unmodulated carrier power 8KW to the antenna. Determine the total

power radiated when modulated to 30%.

3.	A 5 KHz audio tone is used to modulate a 50 MHz carrier causing a frequency deviation of 20 KHz. Determine the modulation index.

4.    Emod(t) = 10 sin (7x108t+5 sin 1100t). Find the modulation index and the carrier frequency.

5.	What is meant by a double superheterodyne receiver?

6.	Mention any two characteristics of receivers.

7.	Noise limiter is used inorder to _________.

8.	State the purpose of using De-emphasis.

9.	Define signal to noise ratio.

10.	A  noise with constant PSD is termed as

a.	Transit noise 	b.	white noise 		c.	shot noise 		d.	resistor noise

PART – B  (5 x 3 = 15 MARKS)

11.	A certain AM wave is observed on an oscilloscope, & it’s maximum & minimum amplitudes are measured as 5 v & 1v respectively. Calculate its Depth of Modulation.

12.	Differentiate Narrow band FM and Wideband FM.

13.	List the disadvantages of ‘Tuned radio frequency receiver’.

14.	Discuss Automatic Frequency Control.

15.	Calculate the overall noise figure of a 3 cascaded amplifier, each stage having a power gain of 10 dB’s and a noise figure of 6 dB’s.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Define and derive the expression for Amplitude modulation, its Power calculation & 	Current relation with necessary waveforms. 										  (12)

b. 	Discuss Need for modulation. 													    (3)

(OR)

17. 	a. 	Explain in detail about Balanced modulator with neat diagram and derive.			  (12)

b. 	State the need for suppression of carriers.										    (3)

18. 	a. 	Define and derive the mathematical expression for FM signal and draw the frequency 	spectrum of FM Wave.														  (12)

b.	A 105.6 MHz carrier is frequency modulated by a 7 KHz sine wave. The resultant FM signal has a frequency deviation of 50 KHz.

i. 	Determine the highest and lowest frequencies attained by the modulated signal.	(1.5)

ii. 	What is the modulation index of the FM wave?								(1.5)

(OR)

[P.T.O]

19.	a. 	Discuss in detail about the Foster seeley discriminator with necessary diagrams.		  (10)

b.	How an FM signal is generated using direct method? Explain.						    (5)

20.	a. 	With a neat block diagram, explain the function of a transmitter system which carries 	different information in their sidebands.											  (12)

b. 	Differentiate High level and Low level transmitter system.							    (3)

(OR)

21.	a. 	With neat diagram, explain about a receiver that uses a mixer atleast once before detection. 	Discuss about the image frequency rejection. 									  (12)

b. 	In a broadcast superheterodyne receiver having no RF amplifier, loaded Q of antenna coupling circuit (at the input to mixer) is 100. If Intermediate frequency is 455 KHz, calculate Image frequency and its rejection ratio at 1000 KHz.					    	    (3)

22.	With a neat block diagram, explain the functioning of FM transmitter system.

(OR)

23.  	a. 	With a neat block diagram, explain the functioning of FM stereo system.			  (12)

b. 	Give a short note on Volume expander.											    (3)

24.	Derive the expression for signal to noise power ratio for DSB-SC system.

(OR)

25.	Derive the expression for signal to noise power ratio for SSB-SC system.