Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	SIGNALS AND SYSTEMS							      	     Time : 3 hours

Subject Code:	12EC214						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mathematically represent an exponential signal.

2.	Is y(t) = x2(t+2)+x(t–2) is causal or non-causal?

3.	Find the Laplace transform of u(t).

4.	What is the final value of the signal corresponding to the Laplace transform Xs=1s−1.

5.	What is meant by aliasing?

6.	A baseband signal has a spectral range that extends from 2KHz to 200KHz. Find the minimum sampling frequency required.

7.	Fourier transform of x(n) is __________.

8.	Z-transform of {1,–2, –1, 3} is __________.

9.	Define step response.

10.	Mention the MATLAB command used to represent frequency response of a system.

PART – B  (5 x 3 = 15 MARKS)

11.	List any three properties of continuous convolution.

12.	State and prove Parseval’s theorem.

13.	Write MATLAB code to perform downsampling.

14.	Find the Z-transform and ROC of x(n) = u(n)–u(n–5).

15.	Compare recursive and non-recursive system.

PART – C  (5 x 15 = 75 MARKS)

16.	The trapezoidal pulse is defined by x(t)=5−t,4≤t≤51,−4≤t≤4t+5,−5≤t≤−40,otherwise.

Perform (a) x(t+2)  	(b) x(t–1)u(t) 	(c) 2x(2t)	(d) x(t) u(t–1) 	(e) x(–t+2)

(OR)

17.	a.	Determine whether the system y (n) = x(n)+ nx(n+1) is causal/ non-causal, static/ dynamic, 	time-invariant/ time variant and linear/ non-linear. Justify your answers.			    (8)

b.	Find the output using convolution for the given signals x[n]={1,0,1,1} and h[n]={1,2,3,1}.

(7)

18.	Find the inverse laplace transform of

a.	X(s)=3s2+8s+6(s+2)(s2+2s+1).

b. 	X(s)=1s(2s2+5s+3).

[P.T.O]

(OR)

19.	a.	The input and output of a causal LTI system is described by the differential 	equationd2y(t)dt2+6dy(t)dt+8y(t)=2x(t). Determine the frequency response and impulse 	response of the system using CTFT.											    (9)

b.	State and explain the following properties of Fourier Transform.

i. Linearity		ii.  Time shifting		iii.  Convolution 							    (6)

20.	With neat diagrams, explain impulse train sampling and reconstruction of Continuous time signal from its samples.

(OR)

21.	a.	List different types of sampling methods and give a comparison among them.		    (8)

b.	Briefly explain discrete time processing of continuous time signals.					    (7)

22.	Find the inverse fourier transform of the following

a.	 X(ej) =1+2e-j -3e-2j + 2e-3j + 4e-4j

b. 	X(ejω)=2j−2j

c. 	X(ej)=e–j(1+ cos)

d. 	X(ej) = e–j4,    π2< || < π

(OR)

23.	a.	Find the impulse response and step response for the system 	y(n)−34y(n−1)+18y(n−2)=x(n).											  (10)

b.	State and prove any two properties in Z-transform.								    (5)

24.	a.	Determine the step responses for the given impulse responses

i. 	h(n) = (n–3) + (n–5)

ii. 	h(n)=14nu(n)															    (6)

b.	Obtain the Direct Form II realization of the system described by the difference equation

y(n)−14y(n−1)−18y(n−2)=x(n)+3x(n−1)+2x(n−2).						    (9)

(OR)

25.	The system function of a discrete time system is H(z)=(1+z−1)4(1−z−1+78z−2)(1+2z−1+34z−2). Realize this system using cascade form.