Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	12EC216						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the two types of sectioned convolutions?

2.	Direct computation of DFT requires ______ complex additions.

3.	What is frequency warping?

4.	Define a Chebyshev Type I filter.

5.	Why is FIR filter stable?

6.	What is the condition for linear phase characteristic of an FIR filter?

7.	Express the fraction  −78 in sign magnitude form.

8.	Define rounding with an example.

9.	What are the steps in an instruction set?

10.	Differentiate general purpose and special purpose processors.

PART – B  (5 x 3 = 15 MARKS)

11.	Obtain the circular convolution of the following sequences.

x(n) = {1,2,1}; h(n) = {1,–2,2}.

12.	Discuss about the frequency transformations in the analog domain.

13.	Draw direct form structure for the system described by the equation

y (n) = 0.5x (n) + 0.9 x (n–1).

14.	Mention the types of errors due to quantization.

15.	What is the need to use adaptive filters?

PART – C  (5 x 15 = 75 MARKS)

16.	An 8-point sequence is given byx(n) = 1 , 2, 3, 4, 4, 3, 2, 1. Compute the 8 – point DFT of x(n)by radix-2 DIF FFT.

(OR)

17.	Perform the linear convolution of the following sequence by overlap add method:

x(n) = {3, –1, 0, 1, 3, 2, 0, 1, 2, 1} and h(n) = {1,1,1}

18.	Design a Butterworth digital IIR filter using the impulse variance method for the following 	specifications:

0.8≤H(ejω)≤1 for 0≤ ω ≤ 0.2π  H(ejω)≤0.2 for 0.6π≤ ω ≤ π

Assume T = 1 sec.

(OR)

[P.T.O]

19.	Design a Chebyshev low pass filter with the specifications using bilinear transformation.

Ap   = 1dB ripple in the pass band           0           0.2

As  = 15 dB	ripple in the stop band        0.3          

20.	Design an ideal low pass filter with the following frequency response using a Hanning window. Find the values of h(n)for N=7. FindH(z).

Hd(ejω) = e−j3ω for −π4≤ ω≤ π40 for π4 ≤ω≤ π

(OR)

21.	Determine the filter coefficients h(n)obtained by sampling for N = 7. FindH(z).

Hd(ejω) = e−j(N−1)ω2 0 ≤ ω <π20 otherwise

22.	a.	Explain quantization noise and error due to truncation and rounding in detail.		  (10)

b.	Discuss the effect on quantization on pole locations for a given system function.		    (5)

(OR)

23.	Explain zero input limit cycle oscillations and derive the equation for dead band.

24.	Draw and explain the architecture of TMS320C5416.

(OR)

25.	a.	Explain the least mean square (LMS) adaptive algorithm in detail.					    (5)

b.	Draw the various blocks in the Harvard architecture and explain them in detail.		  (10)