Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ANTENNAS AND WAVE PROPAGATION			      	     Time : 3 hours

Subject Code:	12EC217								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The radiation resistance of a monopole antenna is ___________.

2.	The effective aperture of a Hertzian dipole antenna is ___________.

3.	Why do we go for antenna arrays?

4.	What is array factor?

5.	What are the independent parameters in design of Rhombic antenna?

6.	___________ is referred to super directive antenna.

7.	The relation between Electric and Magnetic fields is ___________.

8.	__________ is directivity of large rectangular broadside array.

9.	Express the relationship between critical frequency and maximum electron density.

10.	__________are the factors that cause attenuation in ground wave.

PART – B  (5 x 3 = 15 MARKS)

11.	The radiation resistance of an antenna is 72Ω and the loss resistance is 8Ω. What is the directivity if the power gain is 16?

12.	Difference between End fire array and Broadside arrays.

13.	Calculate apex angle and spacing factor for LPDA, Ln+1=10m, Ln=5m, Spacing between the adjacent elements (Sn) is 0.5m.

14.	A Large rectangular aperture with height h=10m & width l=6m. Find aperture area and directivity.

15.	What is sporadic E layer in Ionosphere?

PART – C  (5 x 15 = 75 MARKS)

16.	State & Verify Reciprocity Theorem.

(OR)

17.	Obtain the power radiated and radiation resistance from Halfwave dipole.

18.	Derive the array factor obtained from uniform linear array.

(OR)

19.	Determine the direction of Maxima, Minima and Beam width for end fire Arrays with two point sources.

20.	Derive EMF equation of Loop Antenna.

(OR)

21.	With necessary illustrations explain the radiation characteristics of multi-element log periodic antenna and mention its possible applications.

22.	Derive the expression for radiation from a rectangular aperture treated as an array of Huygen’s source.

(OR)

23.	Illustrate the impedance relation between slot and dipole antenna.

24.	Derive an expression for the refractive index of the ionosphere in terms of the electron number density and frequency.

(OR)

25.	Explain the Structure of ionospheric layers in detail.