Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL COMMUNICATION						      	     Time : 3 hours

Subject Code:	12EC219						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	When aliasing occurs?

2.	Manchester coding is also known as ___________.

3.	Which algorithm is used in adaptive equalization?

4.	What is the major advantage of M-ary signaling?

5.	Minimum bandwidth in DPSK is equal to ____________.

6.	Plot the BFSK signal for the given sequence 1100101.

7.	The weight of the code word 1010001 is ____________.

8.	State the cyclic property of a cyclic code.

9.	The PN sequence length for shift register length m = 4 is __________.

10.	How many numbers of runs this PN sequence - 0011101 has?

PART – B  (5 x 3 = 15 MARKS)

11.	State sampling theorem. Obtain the Nyquist rate for the given signal x(t) = cos (200t).

12.	Illustrate and explain eye pattern.

13.	Compare BFSK and BPSK.

14.	Define entropy. Assume four message bits from a source occurs with probabilities p1=p4=1/8 and p2=p3=3/8, what is the entropy of the source?

15.	State the properties of PN sequence.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain PPM and PWM with necessary waveforms.								    (7)

b.	What is quantization? Explain the types of quantization with necessary diagrams.	    (8)

(OR)

17.	With a neat block diagram explain Delta modulation and discuss the types of quantization errors occurring in it. Also brief the methods to overcome the shortcomings.

18.	Derive the average probability of symbol error of a binary symmetric channel.

(OR)

19.	Describe the base band transmission system. Derive the Nyquist criterion for distortion less base band binary transmission.

20.	Draw the block diagram of QPSK system and explain its operation with signal constellation diagram.

(OR)

21.	Draw the block diagram of MSK system and explain its principle and operation with signal constellation diagram.

[P.T.O]

22.	A discrete memoryless source has an alphabet of five symbols with their probabilities for its output as given here:

| Symbol | s0 | s1 | s2 | s3 | s4 |
| --- | --- | --- | --- | --- | --- |
| Probability | 0.4 | 0.2 | 0.2 | 0.1 | 0.1 |

Compute two different Huffman codes for this source. Hence, for each of the two codes, find

a.	The average code-word length.

b.	The variance of the average code-word length over the ensemble of source symbols.

(OR)

23.	Design a Convolutional encoder of code rate r=1/2 and constraint length of K=3 for the given generator polynomial (i) input-top adder output path is 111 (ii) input-top adder output path is 101.

a.	Encode the message sequence 10011.

b.	Draw the code trellis and state diagram for encoding.

24.	Draw the block diagram of DS-spread spectrum system transmitter and receiver and explain the function performed by each block in detail.

(OR)

25.	Draw the block diagram of frequency hopped spread spectrum system and explain slow frequency hop technique with one example.