Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	MODERN CONTROL SYSTEMS					      	     Time : 3 hours

Subject Code:	12EC220						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define a system.

2.	A tacho generator generates an electrical output proportional to ___________.

3.	When the damping ratio of a second order  system is one it is called __________ damped and its roots are __________.

4.	For a type 1 system the steady state error due to step input is _________.

5.	The slope of a function G(s) = 1/s2 is ___________.

6.	Sketch the approximate polar graph for a Type 1 order 2 system.

7.	Integral Controller reduces steady state error. TRUE / FALSE

8.	What is the necessary condition for stability?

9.	When the system is said to be completely controllable?

10.	Write the characteristic equation for the given matrix.

A=−3−2−1−2

PART – B  (5 x 3 = 15 MARKS)

11.	Find the transfer function of a negative feedback system, with gain G(s) and feedback factor H(s).

12.	Sketch the time response of a second order underdamped system and plot the time domain specifications on it.

13.	Define Resonant peak corner frequency.

14.	Discuss the stability of the system, by finding the impulse response, whose transfer function is A/s+a.

15.	What are the draw backs of transfer function modeling and the advantages of state variable approach?

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the transfer function of the given Signal Flow Graph.

(OR)

[P.T.O]

17.	Write the dynamic equations governing the mechanical system given below. Also obtain the equivalent electrical network using Force Voltage Analogy

18.	Obtain the time response specifications for the unit-feedback system with  G(s)=1s(s+1)

(OR)

19.	A unit-feedback system is characterized by the open-loop transfer function. Obtain the steady state error of the system when subjected to an input given by the polynomial

G(s)=10s(0.1s+1)

r(t)=a0+a1t+a22t2

20. 	A unity feedback control system has GS=10s1+0.4s1+0.1s. Draw the bode plot and determine its gain cross over frequency and phase cross over frequency.

(OR)

21. 	The open loop transfer function of a unity feedback system is given by

GS=1ss+12s+1.

Sketch the polar plot and determine the gain margin and phase margin.

22. 	a.	The characteristic polynomial of a system is S7+5S6+9S5+9S4+4S3+20S2+36s+36=0.      		Determine the location of roots on the s plane and hence the stability of the system.

(10)

b.	List the advantages and disadvantages of P,I and D controllers.						(5)

(OR)

23. 	Sketch the complete root locus of system having

GS=KSS+2S+4

24. 	A linear time invariant system is characterized by the state variable model. Comment on the controllability and observability of the system.

x1'x2'=−100−2x1x2+01u

yt=12x1x2

(OR)

25. 	Obtain the state model of the system whose transfer function is given as

YsXs=10s3+4s2+2s+1.