Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	VLSI DESIGN										      	     Time : 3 hours

Subject Code:	12EC231						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by Twin Tub CMOS?

2.	What are the two common approaches used for Oxidation Process?

3.	Draw the VI characteristics curve of NMOS inverter.

4.	What is Body effect?

5.	According to Mead and Conway design rules, the minimum width of the metal and diffusion features are ___________ and ___________ respectively.

6.	Write the expression for the resistance for uniform slab of conducting material.

7.	The advantage of CMOS domino logic when compared to CMOS logic is_________.

8.	What is pre-charging?

9.	What are the factors to be considered for memory elements?

10.	What are the two logic levels used in PLA’s?

PART – B  (5 x 3 = 15 MARKS)

11.	Name the different levels of abstraction in chip design.

12.	List the second order effects of MOSFET.

13.	Design the basic Boolean functional unit using CMOS transistor logic Y = ((A+B+C).D)’.

14.	Draw the stick diagram for a 2 input CMOS NOR gate.

15.	What is a carry look ahead adder?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the fabrication of n-well and p-well CMOS process with structure and Mask diagram for each step of fabrication.

(OR)

17.	Explain the different steps of Twin tub fabrication giving a schematic for every step.

18.	Starting from the flat band voltage, derive the expression for threshold voltage and current expression for CMOS under three different operating regions.

(OR)

19.	Describe in detail the various regions in a CMOS inverter DC characteristics.

20.	Explain the Design rules for CMOS N-Well Process.

(OR)

21.	Obtain the expression for area capacitance, standard unit of capacitance and calculate the area capacitance of different layers of MOSFET.

22.	Explain two phase clock generator using D flip-flop.

(OR)

23.	Explain in detail a) Dynamic CMOS Logic; b) Clocked CMOS logic; c) Pre-charged domino CMOS logic.

24.	Explain the PLA control Implementation with an example.

(OR)

25.	Explain the structure of a serial-parallel multiplier and the design of basic cell.