Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL DESIGN USING VHDL					      	     Time : 3 hours

Subject Code:	12EC232										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In PLA AND arrays are __________ OR arrays are __________.

2.	Name the different programming technologies.

3.	Define RTL.

4.	Write about emulation.

5.	What is gate array based design?

6.	Define full custom design.

7.	Concurrent signal assignment statements are executed whenever  __________ occur on signals.

8.	Give the syntax of CASE statement.

9.	With example define component declaration statement.

10.	__________ delay is called default delay.

PART – B  (5 x 3 = 15 MARKS)

11.	Design the function using ROM

F1 (A1, A0)=Σm (0, 1)

F2 (A1, A0)= Σm (1,2, 3)

12.	Give the difference between Programmable and Micro programmable controller.

13.	Discuss about standard cell design.

14.	With examples describe in detail about WAIT statements.

15.	By using CASE statement write VHDL code for a 4 x 1 multiplexer.

PART – C  (5 x 15 = 75 MARKS)

16.	Implement the functions using PLA

A(x,y,z)= Σm(0,1,4,5)

B(x,y,z)= Σm(0,2,4,6)

C(x,y,z) = Σm(2,6)

(OR)

17.	a.	Implement the following Boolean logic equations in PLA.

Z=Q2X+Q3X

D1=XQ2Q3

D2=Q2Q3+XQ1+XQ2Q3

D3=Q2X+Q3X															    (5)

b.	Implement the binary to gray code converter (4 bits) using PROM where A, B, C, D are inputs and W, X, Y, Z are outputs.      											  (10)

[P.T.O]

18.	a.	Design a Micro programmed Controller and prepares micro instructions to perform ADD 	macro. Justify your use of instructions.											  (10)

b.	Describe the organization of a hypothetical tiny computer with a neat diagram.		    (5)

(OR)

19.	a.	With the design flow, lists the different operators and sub-programs used in ABEL.     (10)

b.	 Discuss about ROM Simulation.												   (5)

20.	Explain in detail about different types of Gate–array based design with neat diagrams.

(OR)

21.	With neat diagram explain in detail about Xilinx 3000 FPGA and also describe how a 32-bit LUT can realize different functions.

22.	a.	Write a VHDL Program for four bit full adder, using the structural description. Draw its 	basic structure showing the inputs and outputs signals.							  (10)

b.	List the types of data operators available in VHDL with examples.					    (5)

(OR)

23.	a.	Explain in detail about different scalar data types.								  (10)

b.	Describe in detail about signal and constant declarations.  							    (5)

24.	a.	Design a 2 bit magnitude comparator in Behavioral modeling using VHDL (use if-then-	else OR CASE statement).													  (10)

b.	Write VHDL program for full adder using behavioral modeling.					    (5)

(OR)

25.	a.	Explain in detail about conditional signal assignment statement with an example.	  (10)

b. 	Design a D Flip-flop in behavioral modeling in VHDL.							    (5)