Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	MICROWAVE AND OPTICAL COMMUNICATION ENGINEERING													      	     					     Time : 3 hours

Subject Code:	12EC243								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is CGS and MKS?

2.	What is the value of boltzmann constant?

3.	List the characteristics of two cavity klystron amplifier.

4.	How efficiency is calculated in two cavity klystron?

5.	What is RWH theory?

6.	What is VSWR?

7.	State snells law.

8.	What is polarization?

9.	What is a direct and indirect band gap?

10.	What is quantum efficiency?

PART – B  (5 x 3 = 15 MARKS)

11.	Where phase shifters used?

12.	Draw different reentrant cavities.

13.	List the modes of operation of GUNN oscillators.

14.	Explain different types of fibers.

15.	What are the requirements of photodetector in optical communication?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain scattering matrix derivation for circulators.

(OR)

17.	Enumerate passive microwave devices.

18.	An IMPATT diode ha the following parameters

Carrier drift velocity: 				vd=2×107 cms

Drift – region length:				L=6 μm

Maximum operating voltage:  		V0max=100 V

Maximum Operating current:		I0max=200mA

Efficiency:						η=15%

Breakdown voltage:				Vbd=90 V

Compute (a) the maximum CW output power in watts; (b) the resonant frequency in gigahertz.

(OR)

19.	Explain TWT amplifier principle and operation gain.

[P.T.O]

20.	A two - cavity klystron amplifier has the following parameters:

V0=1000 V				R0=40kΩ

I0=25mA				F=3 GHz

Gap spacing in either cavity:							d= 1 mm

Spacing between the two cavities:						L=4 cm

Effective shunt impedance, excluding beam loading:		Rsh=30 kΩ

a.	Find the input gap voltage to give maximum voltage V2.

b.	Find the voltage gain, neglecting the beam loading in the output cavity.

c.	Find the efficiency of the amplifier, neglecting beam loading.

d.	Calculate the beam loading conductance and show that neglecting it was justified in the 	preceding calculations.

(OR)

21.	Explain microwave transistors.

22.	Explain mode theory.

(OR)

23.	Describe optical fibers attenuation dispersion.

24.	Explain physics of light emission and amplification in semiconductors.

(OR)

25.	Describe laser diodes working principle.