Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FUNDAMENTALS OF DIGITAL IMAGE PROCESSING													      	     									      Time : 3 hours

Subject Code:	12EC252								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is gray-level resolution?

2.	How will you represent the four neighbors of the pixel (x, y)?

3.	State the procedure for generating the negative of an input image.

4.	What is the necessity for histogram equalization in image processing?

5.	Define global thresholding.

6.	What is the effect of illumination on the concept of thresholding?

7.	Draw the block diagram of a general compression system.

8.	What is variable length coding?

9.	Write the mathematical equation used for representing the dilation operation.

10.	Why do you perform erosion operation on an input image?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the difference between 4-adjacency, 8-adjacency and m-adjacency?

12.	Discuss the technical aspects of contrast stretching and gray level slicing.

13.	Compare and contrast local thresholding and dynamic thresholding.

14.	Comment briefly on I-frame, B-frame and P-frame in video data.

15.	Differentiate opening and closing morphological operations.

PART – C  (5 x 15 = 75 MARKS)

16.	With neat diagram, explain the various fundamental steps in digital image processing.

(OR)

17.	a.	With schematic diagram, illustrate the RGB color model used for representing the color 	images. 																		(8)

b.	Validate the following statement with neat diagrams and equations. 				(7)

“HSI color model representation is more effective than RGB color model”

18.	How will you use the following arithmetic operations for enhancing the quality of the input images?

a.	Image subtraction. 															(8)

b.	Image averaging. 																(7)

(OR)

19.	With necessary mathematical equations, explore the application of second order derivatives for image enhancement.

20.	Using the concept of masking, how will you detect the points, lines and edges in an input image?

(OR)

[P.T.O]

21.	Explain the region growing, region splitting and merging methodologies for image segmentation.

22.	Encode the alphabets (a1, a2, a3, a4, a5 and a6) with corresponding probabilities (0.1, 0.4, 0.06, 0.1, 0.04 and 0.3) respectively using Huffman coding methodology.

(OR)

23.	Illustrate the methodology of JPEG compression with necessary mathematical equations.

24.	How does the following morphological operations used for digital image processing?

a.	Region filling. 																    (7)

b.	Boundary extraction. 															    (8)

(OR)

25.	Explain the following morphological operations with necessary equations.

a.	Thinning.	 																    (7)

b.	Thickening. 																	    (8)