Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRONICS AND MICROPROCESSORS		      	     Time : 3 hours

Subject Code:	12EC260								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	____________ is the voltage regulator.

2.	What is the efficiency of full wave rectifier?

3.	Define precision.

4.	What are thermistors?

5.	Draw the logic diagram for half adder.

6.	What are Universal logic gates?

7.	Give example for immediate addressing mode in 8085 microprocessor.

8.	List the flags present in 8085 microprocessor.

9.	Define interrupt.

10.	What is meant by DMA?

PART – B  (5 x 3 = 15 MARKS)

11.	State Barkhausen criteria.

12.	Define piezo electric effect.

13.	Convert (F8A.05D)16 to octal and decimal numbers.

14.   List out the registers available in 8085 microprocessor.

15.	Differentiate between Asynchronous and Synchronous data transfer scheme.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With respect to the circuit diagram explain the operation of the Colpitts oscillator.	  (10)

b.	In the Colpitts oscillator C1 = 0.2 micro Farad and C2 = 0.02 micro Farad. If the frequency   of the oscillator is 10Khz, find the value of the inductor. Also find the required gain for oscillation.	                                                                                                                        (5)

(OR)

17.	Derive an expression for the efficiency for a full wave rectifier. What is ripple factor? What is the ripple factor value for Half wave rectifier and full wave rectifier?

18.   Explain the operation of photodiode and phototransistor.	                				       (7+8)

(OR)

19.	Explain the construction and working principle of CRO.

20.	With neat sketches of the logic symbol, explain common logic gates AND, OR, NOT, NAND, NOR with reference to the truth table.

(OR)

21.	Draw the schematic of 4 bit ripple counter and explain its operation with Truth Table and wave forms.

22.	With a neat block diagram, explain the architecture of 8085 in detail.

(OR)

[P.T.O]

23.	Explain different addressing modes of 8085 with examples.

24. Write short notes on:																(7+8)

a.	CRT Terminal			b.	Memory.

(OR)

25.	How the Data are transmitted using DMA? Describe it.