Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	BASIC ELECTRICAL AND ELECTRONICS ENGINEERING

Time: 3 hours

Subject Code:	12EE102						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Kirchhoff’s voltage law.

2.	Two resistances R1 and R2 of values 6  and 10  respectively are connected in parallel, what is the equivalent resistance of the combination?

3.	Define peak factor.

4.	Write the equation of alternating voltage.

5.	Transformer works on the principle of _________.

6.	Write the EMF equation of DC generator.

7.	Draw the symbol of SCR.

8.	Give the truth table for AND gate.

9.	List the uses of Megger.

10.	Mention the two types of moving iron instruments.

PART – B  (5 x 3 = 15 MARKS)

11.	List the limitations of ohm’s law.

12.	Compare two phase and three phase systems.

13.	Explain the principle of dc motor.

14.	Compare OR gate and Ex-OR gate.

15.	List the basic requirements of a transducer.

PART – C  (5 x 15 = 75 MARKS)

16.

Using Kirchoff’s laws, find the current in various resistors in the circuit shown below.

(OR)

17.	Compare series and parallel circuits. Mention its advantages and disadvantages.

[P.T.O]

18.

Determine the equivalent resistance RT by using star-delta transformation and find the total current IT delivered by the voltage source in the following circuit.

(OR)

19.	Derive the expression for RMS value, average value, form factor and peak factor of the sinusoidal alternating quantity.

20.	Explain the working principle and constructional details of D.C Generator.

(OR)

21.	With a neat diagram, explain the construction and working principle of a transformer.

22.	Describe the construction, operation and characteristics of JFET.

(OR)

23.	Explain the action of PN junction diode under forward bias and reverse bias.

24.	Describe the Hall Effect and also write notes on Piezo-Electric transducers.

(OR)

25.	Discuss   moving coil and moving iron instruments.