Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	ENERGY SYSTEMS								      	     Time : 3 hours

Subject Code:	12EE232						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	___________ is to measure beam radiation by using a long narrow tube to collect only beam radiation from the sun at normal incidence.

2.	Draw an equivalent circuit of an ideal solar PV cell.

3.	Define the term cut-in and cut-out velocity in wind turbine.

4.	What are the most favorable sites for installing of wind turbines?

5.	What do you understand by energy farming?

6.	In fuel cell, ___________ energy is converted into _________ energy.

7.	List down the drawbacks of indirect lighting system.

8.	What do you understand by resistance heating?

9.	Bring down the various methods of control the speed in D.C traction motor.

10.	In electric traction, the power is the product of ___________ and __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the declination angle for day number (n) =157.

12.	Comment on the environmental impacts of wind energy.

13.	With neat diagram explain the any one hydrogen production method.

14.	Draw the Rousseau Diagram, from that bring down the equation for average candela of the source of light.

15.	Discuss the characteristics of ideal traction system from owner point of view.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Estimate the hourly global, beam and diffused radiations at New Delhi (28°35’N, 77°12’E) 	between 1450 hours (LAT) on May 15 (A=1106 W/m2, B=0.177, C=0.130).			  (10)

b.	Bring down any five applications of Solar PV system.  							    (5)

(OR)

17.	List down the classification of Solar PV system and explain the Stand-alone System and Gird- Interactive system with neat sketch.

18.	a.	Sketch the diagram of a Horizontal Axis Wind Turbine (HAWT) and explain the functions 	of its main components.														  (10)

b.	What are the most favorable sites for installing of wind turbines?  					    (5)

(OR)

19.	a.	With the help of block diagram, explain the functions of various blocks of a Wind Energy 	Conversion System (WECS).  													    (9)

b.	Explain the mechanism of production of local winds.								    (6)

20.	Describe various energy extraction technologies in vapour –dominated (Dry Steam) energy extraction used with hydrothermal resources.

(OR)

[P.T.O.]

21.	a.	Describe the various methods of production of hydrogen for use as energy carrier.	  (10)

b.	Explain the process of photosynthesis. How much energy is stored through this process? In what range of frequency spectrum is solar light photosynthesis most marked? 		    (5)

22.	a.	With help of neat sketch, explain any two types of bench photometers. 				  (12)

b.	Discuss the applications of flood lighting.										    (3)

(OR)

23.	With help of neat diagram, explain the construction and working of core type induction furnace.

24.	Discuss the various types of traction system.

(OR)

25.	Discuss the special arrangement required for employing regenerative braking in D.C. series motor.