Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	THERMODYNAMICS AND FLUID MACHINERY		      Time : 3 hours

Subject Code:	12EI204								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is a system?

2. 	Define First Law of Thermodynamics.

3. 	What is Heat transfer?

4. 	State Fourier Law.

5. 	List out the classification of boilers.

6. 	Name the types of heat exchanger.

7. 	Define Newton’s law of viscosity.

8. 	What is a pump?

9. 	Define Laminar Flow.

10. 	What is a turbine?

PART – B  (5 x 3 = 15 MARKS)

11. 	State Carnot Theorem.

12. 	Define Conduction, Convection and Radiation.

13. 	How to analyse the heat exchanger?

14. 	Define Specific weight, specific volume and specific gravity.

15. 	Define: (a) steady and unsteady, (b) uniform and non-uniform, (c) Rotational and irrotational flow.

PART – C  (5 x 15 = 75 MARKS)

16. 	A Fluid system contained in a piston and cylinder machine, passes through a complete cycle of four processes. The sum of all heat transferred during a cycle is – 340 kJ. The system completes 200 cycles per min.  Complete the following table showing the method for each item, and compute the net rate of work output in kW.

Process 		Q (kJ/min) 		W (kJ/min) 		ΔE (kJ/min)

1—2 			 0 			      4340 			—

2—3		  42000				 0 				—

3—4		 – 4200 				—		     	   – 73200

4—1 		      — 				—				 —

(OR)

17. 	A Carnot heat engine draws heat from a reservoir at temperature T1 and rejects heat to another reservoir at temperature T3. The Carnot forward cycle engine drives a Carnot reversed cycle engine or Carnot refrigerator which absorbs heat from reservoir at temperature T2 and rejects heat to a reservoir at temperature T3. If the high temperature T1 = 600 K and low temperature T2 = 300 K, determine : (a) The temperature T3 such that heat supplied to engine Q1 is equal to the heat absorbed by refrigerator Q2.(b) The efficiency of Carnot engine and C.O.P. of Carnot refrigerator.

18. 	Explain the general Heat conduction equation in Cartesian coordinates.

[P.T.O]

(OR)

19. 	Calculate the following for an industrial furnace in the form of a black body and emitting radiation of 2500°C :(a) Monochromatic emissive power at 1.2 μm length, (b) Wavelength at which the emission is maximum, (c) Maximum emissive power (d) Total emissive power, and (e) Total emissive power of the furnace if it is assumed as a real surface with emissivity equal to 0.9.

20. 	How heat losses are carried out in boiler and explain the parameters to improve boiler efficiency?

(OR)

21. 	A feed water heater which supplies hot water to a boiler comprises a shell and tube heat exchanger with one-shell pass and two-tube passes. One hundred thin-walled tubes each of 20 mm diameter and length of 2 m per pass are used. Under normal operating conditions, water enters the tubes at 10 kg/s and 17°C and is heated by condensing saturated steam at 1 atm. on the outer surface of the tubes. The convection coefficient of the saturated steam is 10 kW/m2°C. Determine the water exit temperature. Use the following properties of water : Cp = 4.18 kJ/kg°C ; μ = 0.596 × 10–3 Ns/m2 ; k = 0.635 W/m°C and Pr = 3.93.

22. 	Explain in detail about viscosity, Kinematic viscosity, Newton’s law of viscosity? Explain in detail about the types of fluids.

(OR)

23. 	Explain with the neat sketch about the working principle of reciprocating and centrifugal pumps.

24. 	Briefly explain the types of flow.

(OR)

25. 	Explain the working principle of Francis and Kaplan Turbine with the neat sketch.