Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	CONTROL SYSTEM								      	     Time : 3 hours

Subject Code:	12EI210							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Closed loop system differs from open loop system by the presence of ___________.

2.	Thermostat controlled electric heater is an example of open loop system. True/False.

3.	Define time constant.

4.	Damping factor for the under damped second order system is less than ___________.

5.	Test whether the given systen is stable. 5s5 +4s4 +3s3 +s +1=0

6.	A system is said to be stable if ___________.

7.	Phase margin is defined as ___________.

8.	State the type of the system G(s) = 1/s(s+4)

9.	Write the standard equation for state model.

10.	State model can be derived from system transfer function. (TRUE/FALSE)

PART – B  (5 x 3 = 15 MARKS)

11.	List the merits of closed loop system.

12.	Define step input signal.

13.	State the routh stability criterion.

14.	List the frequency domain specifications.

15.	A system is represented by state equation x1x2=2002x1x2+11u(t), Determine state transition matrix.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Obtain the mathematical relation to the given system.    							    (8)

b.	Derive the torque –voltage analogy of the system. 								    (7)

K2                          K1

J2

J1

B2                                B1

(OR)

[P.T.O]

17.	Obtain the model of the system shown in block diagram.

18.	a.	Derive the relation for the step response of the first order system. 					    (8)

b.	Obtain the plot of system G(s)= 1/s+2  for the unit step input.  						    (7)

(OR)

19.	A unity feedback system has a open loop transfer function, G(s) = 10/s(s+2). Find the time domain parameters.

20.	Apply routh criterion and check the stability of given systems:							(8+7)

a.	s4 +2s3 +8s2 + 4s +3=0

b.	s6 +3s5 +5s4 +9s3 +8s2 +6s +4=0

(OR)

21.	Sketch the root locus of system with unity feedback having the open loop transfer function

G(s) = K/ [s(s+1)(s+3)]    [use ordinary graph sheet]

22.	Sketch the bode plot and calculate phase margin for system having the open loop transfer function G(s) = 50/ [(s+1) (s+20] with unity feedback.  [use semilog graph]

(OR)

23.	Sketch the polar plot of the given system with unity feedback. [use polar graph]

24.	Determine the state model of the system given by

Gs=2s+5s+2s+3s+4

(OR)

25.	Obtain the state model in parallel form,   Gs=YsUs=1s2+5s+6.