Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	12EI211							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Draw the circuit of CMOS INVERTER.

2. 	Emitter coupled logic belongs to __________family.

3. 	(0.10110)2 = (_________)10.

4. 	A + A’ = __________.

5. 	F= A’B + AB’ is ____________ gate.

6. 	Number of flip – flops required to realize a MOD 7 counter is ____________.

7.  	In a sequential circuit the present output depends on both the _________ and __________.

8.  	Draw the state diagram for the 2 bit counter.

9.	PAL consists of ____________.

10. 	Fan-out defines the maximum number of ____________ that can be connected to the digital circuit.

a. 	load gate Inputs		b. 	load gate outputs		c. 	Channels	d. 	Noise

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the diagram of 2 input CMOS NOR gate.

12.  Prove (A + B) (B + C) (C + A) = (A+B)(B+C)(C+A).

13.	Draw the block diagram of 4x1 multiplexer.

14.	List the advantages of synchronous counter over that of asynchronous counter.

15. 	Differentiate static RAM and dynamic RAM.

PART – C  (5 x 15 = 75 MARKS)

16. 	Draw the TTL NAND gate circuit with Totem pole output and explain its operation.

(OR)

17.	a.	List and explain CMOS characteristics.											   (5)

b.	With the help of circuit schematic, explain the operation of CMOS NOR gate.		  (10)

18.	a.	Simplify the function using K – map f (a, b, c, d) =  m ( 1, 5, 6, 7, 11, 12, 13, 15)

Also identify the redundant term if any.

(OR)

19.	a.	Express the base – 10 number a = 12.1510 in IEEE 754 floating – point standard using 	single precision.																  (10)

b.	State De-Morgan’s theorems.													    (5)

20.	a.	Design a binary to gray code converter for 3 bits and implement the same using gates. (8)

b.	Draw the Truth Table for Half adder and obtain the expressions for SUM and carry and 	realize the same using logic gates              										    (7)

(OR)

[P.T.O]

21.	a.	Implement the following function using Logic gates

f (x,y, z) =  m (1, 2, 3, 4, 5, 7). 												    (8)

b.	Realize a 8 : 1 multiplexer using two 4 : 1 multiplexers.							    (7)

22.	Design a MOD12 counter using D Flip-Flop.																								(OR)

23.	Draw the schematic of universal shift register and explain its operation.

24. 	With a block diagram explain a PLA and PAL device.

(OR)

25. 	Explain the architecture of FPGA.