Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	INSTRUMENTATION AND CONTROL IN PETROCHEMICAL INDUSTRIES											     Time : 3 hours

Subject Code:	12EI236								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name any two by-products obtained from the Petroleum refinery process.

2.	Define Dew Point.

3.	What is the need of stirrer in the chemical reactor?

4.	Sketch the typical drying curve.

5.	Degrees of freedom = ________ - ________.

6.	Draw the P&ID diagram of Steam and Condensate.

7.	What is multiple effect evaporator?

8.	How many effects of evaporation are used in paper industry to concentrate the Kraft liquor?

9.	What is the electrode potential of the cyanide waste solution?

10.	Define ORP.

PART – B  (5 x 3 = 15 MARKS)

11.  	Write short notes on the basic distillation equipment.

12.  	Differentiate batch dryers and continuous dryers.

13.  	Draw the Piping & Instrumentation diagram of steam heater and list all the variables in that.

Also determine the degrees of freedom of steam heater.

14.  	The pure water boils at 212∘Fat one atmosphere and a 35% Sodium Hydroxide solution  	boils at about 250∘F at one atmosphere. Determine the Boiling point rise.

15.  	List the various treatments employed to the influent to avoid water pollution.

PART – C  (5 x 15 = 75 MARKS)

16. 	Elaborate the working of various pressure control methods of distillation column with    necessary diagrams.

(OR)

17. 	a.	Explain about the column feed temperature control.					      			    (5)

b. 	Illustrate the working of the various control strategies involved in the Reboiler.	     	  (10)

18.	a. 	Describe the pressure control in a chemical reactor by throttling the flow of vent gas.		    (7)

b. 	Elucidate about the cascade temperature control with heating and cooling ability in chemical reactors.															    (8)

(OR)

19. 	Discuss in detail about the operation of various batch dryers.

20. 	Discuss the Instrumentation and control involved in the condenser.

(OR)

21. 	Elaborate the different types of control implicated in Liquid-to-Liquid Heat exchanger.

[P.T.O]

22.  What is an Evaporator? Classify the types of Evaporators. Explain all the different types of  		Evaporators in detail with necessary diagrams.

(OR)

23.  a. 	Explain the common methods used to measure the product density in an evaporator control

scheme.																		    (6)

b. 	Demonstrate the measurement and control of absolute pressure in an evaporator.        (9)

24. 	Exemplify the concept of Chemical Oxidation in both batch and continuous process with relevant chemical equations.

(OR)

25.	a. 	Explain the process of waste water treatment using Chemical Reduction method.		    (8)

b. 	Describe the schematic control system of an acid waste neutralization system.		    (7)