Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	INSTRUMENTATION AND CONTROL FOR AIRCRAFT													      	     								      Time : 3 hours

Subject Code:	12EI240								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In _________ display the information is presented in numerical value.

2.	What is the alternate system, in case of failure of pitot static system?

3.	Name the instruments that work based on the air data given by pitot tube.

4.	List the advantage of heating element in the Pitot Static tube.

5.	List the limitations of a free gyroscope.

6.	Write the principle of gyro horizon.

7.	In high-performance aircraft the measures of fuel quantity in terms of _________ fuel gauging.

8.	List the types of thermocouple used in aircraft.

9.	The power developed by a turbine engine is dependent on _________ and _________.

10.	Define gauge pressure.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the significance of coloured markings applied to the dials of Certain instruments?

12.	What are the principal components and instruments which comprise an aircraft pitot-static system?

13.	Define the two fundamental properties of a gyroscope.

14.	Explain how the Wheatstone bridge circuit may be utilized for the measurement of temperatures.

15.	Differentiate direct reading and remote indicating pressure gauge.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the construction and operation of light emitting display.

(OR)

17.	What do you understand by the term "head-up display"? With neat diagrams describe how required basic flight data is displayed to a pilot.

18.	Describe the construction and operation of airspeed indicator.

(OR)

19.	Explain in detail the operation of an instantaneous vertical speed indicator.

20.	Explain with a neat diagram how a rate gyroscope may be utilized to sense both banking and rate of turn.

(OR)

21.	With neat diagram, describe the principle of directional gyro.

22.	Explain the fundamental operating principle of Bourdon tube used for measurement of   pressure in aircraft.

(OR)

23.	Draw a circuit of a typical capacitance-type fuel-quantity indicating system. Explain the operating principle.

24.	Describe the construction of a thermocouple probe used for turbine engine exhaust gas temperature measurement.

(OR)

25.	With neat sketch explain engine vibration indication system.