Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	INSTRUMENTATION AND CONTROL SYSTEMS      	     Time : 3 hours

Subject Code:	12EI254								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give examples for integrating instruments.

2.	What is random error?

3.	List the devices used for low pressure measurement.

4.	Which transducer converts heat energy into electrical energy?

5.	Write the application of hot-wire anemometer.

6.	Electromagnetic flow meter is not suitable for which kind of fluids?

7.	In signal flow graph, a path which originates and terminates at the same node is called as __________.

8.	What is a transfer function?

9.	Define the term “rise time” in a second order system.

10.	Find the order and type for system having G(s) =1/s3 (s+1).

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between precision and accuracy.

12.	What is Negative Temperature Coefficient?

13.	Write the principle of mechanical torsion meter.

14.	What is the condition of stability of a system?

15.	Define gain margin and phase margin.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the Fundamental Process of measurement and describe generalized measurement system with example.

(OR)

17.	What are the different errors encountered in measurements? Explain with examples and the measures taken to minimize these errors.

18.	Draw the schematic diagram of a radiation pyrometer and explain its operation in 	measurement of temperature. Mention a few applications.

(OR)

19.	With necessary diagrams, explain the construction and working of RTD and thermocouple.

20.	With a neat diagram explain the construction and working of ultrasonic flow meter.

(OR)

21.	Explain the principle and characteristics of LVDT and discuss how LVDT can be used for Displacement measurements.

22.	Explain the principle and working of a pneumatic system.  Differentiate it from hydraulic system.

[P.T.O]

(OR)

23.	With suitable examples explain about open loop and closed loop control systems.

24.	Construct Routh array and determine the stability of the system represented by the characteristic equation, s6 + 2s5 + 8s4 + 12s3 + 20s2 + 16s + 16 = 0.

(OR)

25.	A unity feedback control system hasG(s)=40s(s+2)(s+5). Draw the Bode plot. Find gain margin and phase margin.