Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	THERMODYNAMICS								      	     Time : 3 hours

Subject Code:	12FP202						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Specific Volume is the reciprocal of ___________.

2.	In ___________ system energy is exchanged with surrounding but not mass.

3.	First law of thermodynamics is based on law of conservation of ___________.

4.	Constant Volume process is called ___________.

5.	The measure of unavailable energy is called ___________.

6.	Refrigerator works on the principle of ___________.

7.	The expansion of fluid through a throttling valve is an ___________ Process.

8.	For ideal gases the intermolecular force of attraction is ___________.

9.	Partial molar free energy is called ___________.

10.	Escaping tendency is called ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Zeroth law of Thermodynamics.

12.	Why there is a deviation from the ideal behavior for the gases?

13.	What is the application of Maxwell’s thermodynamic relation?

14.	Define entropy.

15.	Define partial molar properties.

PART – C  (5 x 15 = 75 MARKS)

16.	What is the difference between state function and path function? Prove that work is a path function and energy is a state function.

(OR)

17.	a.	What is the difference between closed, open and isolated system.					    (9)

b.	Calculate the density of the chlorine gas 503.15 K and 152 bar using  ideal gas law.	    (6)

18.	a.	A stationary mass of gas is compressed without friction from an initial state of 0.3m3 and 	0.105 MPa to a final state of 0.15 m3 and 0.105 MPa, the pressure remaining constant 	during the process. There is a transfer of 37.6 KJ of heat from the gas during the process.

How much does the internal energy of the gas change.							    (9)

b.	Derive the expression for the change in enthalpy for flow process using first law of 	thermodynamics.																    (6)

(OR)

19.	What is equation of state and explain various equation of state for real gases.

20.	a.	What is Kelvin-Planck and Clausius statement of second law of thermodynamics?	    (6)

b.	Explain Carnot cycle with various process involved and get the expression for its 	efficiency.																	    (9)

(OR)

[P.T.O]

21.	a.	A cyclic heat engine operates between source temperature of 800°C and a sink of 	temperature 30°C. What is the least rate of heat rejection per KW net output of the engine?																				    (5)

b.	A room is maintained at 27°C while the surroundings are at 2°C. The temperatures of the inner and outer surfaces of the wall (K = 0.71 W/mK) are measured to be 21°C and 6°C respectively. Heat flows steadily through the wall 5m x 7m in cross section and 0.32 m in thickness. Determine i) the rate of heat transfer through the wall, ii) the rate of entropy generation in the wall and iii) the rate of total entropy generation with this heat transfer process.																		  (10)

22.	a.	Write short notes on entropy disorderness and absolute entropy.					    (6)

b.	Derive the entropy change for an ideal gas.										    (9)

(OR)

23.	Explain T-S diagram for pure substance in detail.

24.	Explain the P-V diagram for the pure substances in detail.

(OR)

25.	Explain h-s diagram or mollier diagram for a pure substances.