Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	HEAT AND MASS TRANSFER						      	     Time : 3 hours

Subject Code:	12FP209						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the modes of Heat Transfer?

2.	What is the driving force for Heat Transfer?

3.	Heat transfer rate of natural convection is ________ than forced convection.

4.	Unit of convective heat transfer coefficient is ________.

5.	Heat transfer through vacuum can occur only by ________.

6.	________ is defined as perfect emitter or absorber.

7.	Which is the simplest non-contact type heat exchanger?

8.	The acronym LMTD stands for ________ ________ ________ ________.

9.	What is the driving force for diffusion?

10.	What do you mean by mass fraction?

PART – B  (5 x 3 = 15 MARKS)

11.	Define thermal conductivity.

12.	Write a note on forced convection.

13.	State Stefan–Boltzmann law.

14.	Give the classification Heat exchangers.

15.	State Fick’s law of diffusion.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive an expression for heat conduction in composite rectangular wall in series.		  (10)

b.	Derive an expression for heat transfer through a tubular pipe.						    (5)

(OR)

17.	a.	In an experiment to measure thermal conductivity of meat beef was made into square 	section block 5 X 5 Cm and 1 cm thick. The edges of block were insulated and the heat 	was supplied continuously to one face of block at a rate of 0.80 watts. The temperatures of 	each face were found to be 28.5 °C and 23.3 °C. Calculate its thermal conductivity.  	    (5)

b.	A furnace is made out of the following materials in order from A to C. The temperatures are 400°C and 40°C in inside and outside respectively. Find the rate of heat loss per square meter and temperatures at junction of steel and brick wall and at junction of brick and sil-o-cel brick walls.																  (10)

18.	Describe in detail about Boiling heat transfer.

(OR)

19.	Explain in detail about different types of convective heat transfer.

[P.T.O]

20.	a.	Explain in brief about i) Absorption ii) Reflection and iii) Transmission of electromagnetic 	radiation.																	  (10)

b.	Determine the total emissive power of a black body at 1027◦C.						    (5)

(OR)

21.	In a canning process,  100 cans per minute  are  to be heated from 60oc  to 100  °C,  and  demoistured  of the  adhering  moisture  to  the  cans.  Each can contains on average 0.3 m1 of water adhering to it.  The cans have an emissivity of 0.7 and an average area of each can is 0.047  m2 ,  and  its  weight is  100g.  The  specific  heat  of can  material  is  0.25 kJ/kg°C. Calculate the temperature of the oven considering the radiative heat transfer only.

Data:

22.	Describe with a neat sketch the construction and working of plate Heat exchanger.

(OR)

23.	Explain the working of 1-2 shell and tube heat exchanger with construction details.

24.	Describe the principles and theory of diffusion.

(OR)

25.	a.	Explain in detail about Whitman’s two-film theory regarding interphase mass transfer.

(10)

b.	Write a note on factors affecting convective mass transfer.							    (5)