Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	STORAGE ENGINEERING							      	     Time : 3 hours

Subject Code:	12FP235						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Roundness is a measure of __________ of the solid material.

2.	The ratio of the density of the material to the density of water is called __________.

3.	Specific heat of a product will tell about the amount of __________ required to change the temperature of the product.

4.	The two methods of expressing the moisture content is __________.

5.	__________ is an example for traditional storage structure.

6.	Methyl bromide is used for __________ during storage.

7.	Carbon dioxide can be used as an __________ for storage of grains.

8.	Trogoderma granariam is generally called as __________.

9.	The effect of low oxygen content on controlling the pests largely depends on the __________.

10.	MAP is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define terminal velocity.

12.	Define equilibrium moisture content.

13.	What are the sources of infestation in grain storage?

14.	What are rodenticides?

15.	What are the types of modified atmospheric storage?

PART – C  (5 x 15 = 75 MARKS)

16.	How the thermal conductivity of the grain is determined and what is its application?

(OR)

17.	Explain structure and composition of food grains.

18.	What are constant and falling drying rate periods? Derive the expression for time of drying.

(OR)

19.	Write short note on

a.	Fluidized bed dryers.															    (5)

b.	Rotary dryer.																	    (5)

c.	Tray dryer.																	    (5)

20.	What are the important characteristics of bulk materials to be considered for storage and discuss the importance of bulk density, true density and angle of repose?

(OR)

21.	How the pressure distribution in storage vessel is calculated and what is its importance?

22.	What is the difference between fumigation and aeration and what is the need for fumigation?

(OR)

23.	What are the direct and indirect damage during storage of food grains?

[P.T.O]

24.	How effective is modified atmosphere storage for cereal grain storage? Explain with example.

(OR)

25.	Write short note on

a.	Airtight storage																(7)

b.	Dust hazards																	(8)