Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ENGINEERING THERMODYNAMICS				      	     Time : 3 hours

Subject Code:	12ME213										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is PMM 1? Why is it impossible?

2.	Distinguish between open and closed systems.

3.	What is a thermal energy reservoir?

4.	Define availability.

5.	What do you mean by triple point?

6.	State the phase rule of pure substances.

7.	Write the Vander wall’s equation for a real gas.

8.	State the Daltons law of partial pressure.

9.	What is dew point temperature?

10.	What is psychrometer?

PART – B  (5 x 3 = 15 MARKS)

11.	Write SFEE and reduce it to turbine and compressor.

12.	State Kelvin-Planck and Clausius statements of second law of thermodynamics.

13.	Saturated steam has entropy of 6.76 kJ/kg K. What are its pressure, temperature, specific volume, and enthalpy?

14.	Differentiate ideal and real gases.

15.	If the relative humidity of air is 60% at 30˚C, what is the partial pressure of water vapour?

PART – C  (5 x 15 = 75 MARKS)

16.	A piston and cylinder machine contains a fluid system which passes through a complete cycle of four processes. During a cycle, the sum of all heat transfer is 170kJ. The system completes 100 cycles per minutes. Complete the following table showing the method for each item, and compute the net rate of work output in kw.

| Process | Q(kj/min) | W (kj/min) | E(kj/min) |
| --- | --- | --- | --- |
| a-b | 0 | 2,170 |  |
| b-c | 21,000 | 0 |  |
| c-d | -2,100 |  | -36,600 |
| d-a |  |  |  |

(OR)

17.	In a steady flow apparatus, 135 kJ of work is done by each kg of fluid. The specific volume of the fluid, pressure and velocity at the inlet are 0.37 m3/kg, 600 kPa and 16m/s. The inlet is 32m above the floor, and the discharge pipe is at floor level. The discharge conditions are 0.62 m3/kg, 100kPa and 270m/s. The total heat loss between the inlet and discharges is 9 kJ/kg of fluid. In flowing through this apparatus, does the specific internal energy increase or decrease, and by how much?

[P.T.O]

18.	a.	Establish the inequality of Clausius.											(10)

b.	Prove that entropy is a property of the system. 									  (5)

(OR)

19.	Two reversible heat engines A and B are arranged in series. A rejects heat directly to B. Engine A receives 200 kJ at a temperature of 421˚C from the hot source while engine B is in communication with a cold sink at a temperature of 5˚C. If the work output of A is twice that of B, find (a) Intermediate temperature between A and B (b) Efficiency of each engine (c) Heat rejected to the sink.

20.	Draw and explain the phase equilibrium diagram for a pure substance on T-S plot with relevant constant property lines.

(OR)

21.	Steam initially at 1.5 MPa, 300˚C expands reversibly and adiabatically in a steam turbine to 40˚C. Determine the ideal work output of the turbine per kg of steam.

22.	a.	Draw and explain the generalized compressibility chart.							  (10)

b.	State the assumptions made in kinetic theory of gases.							   (5)

(OR)

23.	Find the pressure of nitrogen gas at T= 175 K and v=0.00375 m3/kg. Using (a) Ideal gas equation (b) Vander waals equation of state. Take the molecular weight of nitrogen as 28.013 and T critical =126.2 K and P critical =3.39 MPa.

24.	Air at atmospheric condition of 1.0132 bar has a dry bulb temperature of 32˚C and a wet bulb temperature of 26˚C. Without using psychrometric chart, calculate the following: (a) partial pressure of water vapour (b) Specific humidity (c) Dew point temperature (d) Relative humidity (e) Degree of saturation (f) Density of the air in the mixture (g) Density of the vapour in the mixture (h) enthalpy of the mixture.

(OR)

25.	Atmospheric air with dry bulb temperature of 40˚C and wet bulb temperature of 20˚C is cooled to 15˚C without changing its moisture content calculate the following (a) Original relative humidity (b) Final relative humidity (c) Final wet bulb temperatures (d) Heat removed.