Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	THERMAL ENGINEERING II						      	     Time : 3 hours

Subject Code:	12ME215						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The inside diameter of the cylinder is called __________.

2.	List any two effects of detonation.

3.	Define air standard efficiency.

4.	The mean effective pressure of diesel engine is given by __________.

5.	Define Mach number.

6.	Give any two applications of convergent nozzles.

7.	List any two advantages of turbojet engines.

8.	Classify rocket engines.

9.	An opening through which air is supplied to the treated space is called __________.

10.	List few applications of air conditioning.

PART – B  (5 x 3 = 15 MARKS)

11.	Classify Internal Combustion engines.

12.	Plot the P-V and T-S diagram of Otto cycle.

13.	Define the term Fanno flow.

14.	Classify gas turbines.

15.	What are the variables involved in estimation of load?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain briefly battery or coil ignition system and magneto-ignition system with neat sketches.

(OR)

17.	Discuss briefly wet sump lubrication system with neat sketch.

18.	In an engine working on dual cycle, the temperature and pressure at the beginning of the cycle are 90°C and 1bar respectively. The compression ratio is 9. The maximum pressure is limited to 68bar and total heat supplied per kg of air is 1750 kJ. Determine a) Pressure and temperature at all salient points b) Air standard efficiency.

(OR)

19.	An engine of 250mm bore and 375mm stroke works on Otto cycle. The clearance volume is 0.00263m3. The initial pressure and temperature are 1 bar and 50°C. If the maximum pressure is limited to 25bar, find the following: (a) The air standard efficiency of the cycle (b) The mean effective pressure for the cycle. Assume the ideal conditions.

20.	A gas at a pressure of 70 Kpa and temperature of 295 K enters a combustion chamber at a velocity of 75 m / sec. The heat supplied in a combustion chamber is 1250 KJ / Kg. Determine the Mach number, pressure and temperature of gas at exit. Take γ=1.3 and R = 0.46 KJ / Kg K.

(OR)

[P.T.O]

21.	A conical air diffuser has an inlet diameter of 40cm and exit diameter of 80cm. Air enters the diffuser with a static pressure of 200KPa, static temperature of 37oC. The average velocity of flow at the inlet of the diffuser is 265m/s. Calculate a) mass flow rate b) the properties at the exit section and c) force exerted on the diffuser walls.

22.	In an air – standard regenerative gas turbine cycle the pressure ratio is 5. Air enters the compressor at 1 bar, 300K and leaves at 490k.The maximum temperature in the cycle is 1000K.Calculate the cycle efficiency, given that the efficiency of the regenerator and the adiabatic efficiency of then turbine are each 80%. Assume for air, the ratio of specific heats is 1.4.

(OR)

23.	Explain with a neat sketch, the principle of operation of a turbojet engine and state its advantages and disadvantages.

24.	Explain different types of air distribution systems with neat sketch.

(OR)

25.	Explain the components and working of a window type package air-conditioning system with a neat sketch.