Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FINITE ELEMENT ANALYSIS						      	     Time : 3 hours

Subject Code:	12ME234						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the 3 stages of Finite element analysis according to computer implementations.

2.	List the application of finite element in thermal engineering.

3.	Define aspect ratio.

4.	Write the types of Boundary conditions.

5.	Define Natural co-ordinate system.

6.	What do you mean by the term c0 continuity?

7.	Define serendipity element.

8.	Define plane stress.

9.	Define ideal fluid.

10.	Write the governing differential equation for a steady state heat transfer in 1D composite wall.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between boundary value problem and initial value problem.

12.	What is node? Also list its functions.

13.	Why linear triangular element is often called as Constant strain triangular (CST) element?

14.	Define isoparametric element.

15.	State Darcy’s law.

PART – C  (5 x 15 = 75 MARKS)

16.	Consider the differential equation for the problem as d2ydx2+300x2=0; 0 £	x £ 1  with the boundary conditions  y(0) = 0 and y(1) = 0. Find the solution of the problem using a one coefficient trial function as y=a1x(1–x3). Use Galerkin’s method, point collocation method, sub domain method, least square method.

(OR)

17.	Find the maximum deflection for a clamped beam subjected to uniformly distributed load throughout its length. Use point collocation method. Assume trial function as

y=a(x5–2Lx4+L2x3)

18.	Explain the step by step procedure of FEA and discuss the factors to be considered while discretization.

(OR)

19.	Calculate the nodal displacements and nodal forces for the bar loaded as shown in the figure.

6kN

12kN/mm (K1)

8kN/mm (K2)

[P.T.O]

20.	Derive the shape functions for 2D linear triangular element.

(OR)

21.	With example, explain convergence criteria.

22.	Derive shape functions for nine noded quadrilateral element.

(OR)

23.	Calculate the element stresses for the plane stress element shown in figure. The nodal displacements are u1 = 2 µm ; v1 = 	1 µm, u2 = 0.5 µm ; v2 = 1.5 µm, u3 = 1.2 µm ; v3 =2.8 µm.  Young’s modulus = 210GPa and Poisson’s ratio = 0.25.

x

y

1

(10, 8)

2 (15, 5)

3 (18, 12)

24.	Determine the temperature distribution in a one dimensional fin with the physical properties given in figure. The fin is rectangular in shape and is 8cm long, 4cm wide and 1cm thick. Assume that convection heat loss occurs from the right end of the fin.  (Use 2 element idealization)

φf=20∘C, h=0.1Wattscm2 ∘C             k=3Wattscm ∘C

L=8 cm

80C

4cm

1cm

(OR)

25.	Derive the element stiffness matrix for one dimensional heat transfer problem in thin fins.