# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

# Subject Title: 	ADVANCED THERMODYNAMICS AND HEAT TRANSFER												       								Time : 3 hours

# Subject Code: 	12ME311										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

(Use of approved Thermodynamics property tables, HMT data book are permitted)

1.	Compulsory:

Air enters a compressor at 1 bar, 30oC, which is also the state of the environment. It leaves at 3.5 bar, 141oC and 90 m/s. Neglecting inlet velocity and potential energy effects, determine (a) whether the compression is adiabatic or polytropic, (b) if not adiabatic, the polytropic index, (c) the isothermal efficiency (d) the minimum work input and irreversibility, and        (e) the second law efficiency. Take Cp of air = 1.005 kJ/kg K.

2.	A closed, rigid tank filled with water vapor, initially at 20 MPa, 520oC, is cooled until its temperature reaches 400oC. Using the compressibility chart, determine

(a) the specific volume of the water vapor in m3/kg at the initial state.

(b) the pressure in MPa at the final state.

(OR)

3.	a. 	Derive generalized first and second Tds relations according to the mathematical 	conditions for exact differentials. 											      (10)

b. 	Derive Maxwell’s equations.												      (10)

4.	Briefly discuss the criterion of chemical equilibrium for the process in an adiabatic system and also at a specified pressure and temperature.

(OR)

5.	Determine the adiabatic flame temperature of the stoichiometric butane-air mixture at 298 K, 0.1 MPa, assuming no dissociation of the products for the following two cases: (a) evaluating Cp of each species at 298 K and (b) evaluating the Cp of each species at 2000 K.

6.	Water is to be boiled at atmospheric pressure in a mechanically polished stainless steel pan placed on top of a heating unit. The inner surface of the bottom of the pan is maintained at 108oC. If the diameter of the bottom of the pan is 30 cm, determine (a) the rate of heat transfer to the water and (b) the rate of evaporation of water.

(OR)

7.	The condenser of a steam power plant operates at a pressure of 7.38 kPa. Steam at this pressure condenses on the outer surface of horizontal pipes of through which cooling water circulates. The outer diameter of the pipes is 3 cm, and the outer surfaces of the pipes are maintained at 30oC. Determine (a) the rate of heat transfer to the cooling water circulating in the pipes and (b) the rate of condensation of steam per unit length of a horizontal pipe.

8.	Heat transfer from the walls of the tube to the flowing fluid is related by the formula.. The Reynolds number and the Prandtl number of the fluid flowing inside the tube are given as 50000±500 and 4.71±0.25. Determine the nominal value of the Nusselt number and specify the suitable uncertainty for the same. Also express the uncertainty as a percentage.

(OR)

9.	a. 	Explain in detail about various methods of cooling available for electronic equipment.																				      (12)

b. 	Briefly discuss the working principle of heat pipe.								(8)