Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	CIRCUIT THEORY									      	     Time : 3 hours

Subject Code:	12MT201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Node voltages.

2.	Mention different types of Kirchoff’s law.

3.	Draw the Norton’s equivalent circuit.

4.	State the reciprocity theorem.

5.	Give an example of an electrical device which has the mutual inductance property.

6.	Write the expression of Q-factor of a Capacitor.

7.	Write the mathematical expression for unit step.

8.	What is the time constant of RC series circuit?

9.	Define graph of a network.

10.	What do you mean by tree?

PART – B  (5 x 3 = 15 MARKS)

11.	State Kirchoff’s laws.

12.	State and explain superposition theorem.

13.	Derive the expression for resonant frequency.

14.	Distinguish between the forced response and free response of a circuit.

15.	Write about Tie-set Schedule.

PART – C  (5 x 15 = 75 MARKS)

16.	Write a mesh equation for a network shown below and find the current flow through each resistance.

(OR)

17.	Write nodal equations for the circuit shown.

[P.T.O]

18.	Determine the maximum power delivered to the load in the circuit.

(OR)

19.	Find the Norton equivalent of the circuit shown in figure.

20.	Obtain the resonant frequency, Q – factor, band width and the voltage across the capacitor at resonance for the series R – L – C circuit having R = 7.5 Ω, L = 6μH and C = 40 PF, with a supply voltage of 0.5 volts.

(OR)

21.	For the circuit shown below determine the impedance at resonant frequency, 10Hz above resonant frequency, 10Hz below resonant frequency.

22.	In an RL series circuit with R = 100 ohms and L = 1 Henry, determine the complete solution for the current, when switch is closed at t=0.  Applied voltage is v(t)=200sin[500t].

(OR)

23.	An RC series circuit with R = 10 ohms and C = 100 microfarads has an initial charge Q0 = 2 x 10-2 Coulombs. Find the transient current if the switch is closed at t = 0.

24.	For the network shown below write the tie-set matrix and determine the KVL equation and loop currents.

(OR)

25.	a.	Explain the concept of duality.	                   										    (7)

b.	Obtain the dual network for the network shown in the figure.   						    (8)