Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	THIN FILMS TECHNOLOGY FOR ENGINEERS	      	     Time : 3 hours

Subject Code:	12PH209								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Insulators can be deposited by __________ sputtering.

2.	__________ pumps use no oil and operates like jet engines.

3.	In MBE the source material is   __________ from effusion cells.

4.	Evaporation boat is made up of __________ materials.

5.	What type of stress is applied to the film due to differential thermal expansion of the substrate?

6.	Adsorption that occurs by forming a covalent or ionic bond with a surface atom is known   as

__________.

7.	In epitaxial growth __________ is a measure of structural compatibility between different materials.

8.	A material in which the crystallites are nearly aligned in all 3 dimensions is called a __________.

9.	Thin film capacitor consists of bottom electrode, __________ and top electrode.

10.	Gas sensor is a sub class of __________ sensors.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the importance of vacuum coating in thin film technology.

12.	Mention the different steps involved in sol-gel process.

13.	What is the use of Scherrer’s equation?

14.	Compare polycrystalline, amorphous and single crystalline material based on texture.

15.	What is a thin film transistor? List the basic types of thin film transistors.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain with suitable diagram the instrumentation, working principle, advantages and drawbacks of diffusion pump.

(OR)

17.	With a neat sketch explain in detail the construction and working of McLeod gauge.

18.	With a neat sketch, explain the principle, kinetics and working concept of CVD.

(OR)

19.	With suitable diagram explain the construction and working of molecular beam epitaxy.

20.	With appropriate figures, explain in detail, the different types of lattice mismatch.

(OR)

21.	a.	Define interface. With a neat sketch explain the various interfacial processes.		  (12)

b.	Briefly explain the main reasons for loss of film adhesion.                              		   (3)

22.	Define Hall Effect and describe how this technique can be used to find the electrical properties of thin films.

(OR)

[P.T.O]

23.	Define Bragg’s law. Describe how the structure of a material and the particle size are determined by X- ray diffraction studies.

24.	a.	What are the various factors that should be considered during the design of thin film 	capacitors?																	  (10)

b.	Briefly discuss the steps in fabrication of a thin film solar cell.						    (5)

(OR)

25.	a.	Explain in detail, the design rules for fabrication of thin film resistors.				  (10)

b.	Outline the basic steps in fabrication of MEMS device.							    (5)