# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

# Subject Title: 	RADIATION TREATMENT PLANNING			       Time : 3 hours

# Subject Code: 	12PH323									       	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1. 	Compulsory:

a.	Describe the various volumes of interest as defined in ICRU 62.				      (10)

b.	MRI alone cannot be used for radiotherapy simulation and planning. Why? 			(2)

c.	Write a short note on wedge filters.												(6)

d.	Define Beam’s eye view.														(2)

2. 	a.	Explain the construction and working of a typical therapy X-ray tube. 			      (10)

b.	Explain the configuration of modern linac with neat diagram. 					      (10)

(OR)

3.	a.	Compare the telecobalt machine with the linac.									(4)

b.	Discuss the major steps involved with a CT simulator. 							(6)

c.	Explain the construction and working of betatron and cyclotron.				      (10)

4. 	a.	A source is producing an intensity of 456 R/hr at one foot from the source. What would

be the distance in feet to the 100, 5, and 2 mR/hr boundaries?						(5)

b.	Discuss the variation in PDD with depth at various photon energies.				(5)

c.	Derive the relationship between PDD and TAR.		            					      (10)

(OR)

5.	a.	Draw the Megavoltage X ray beam profiles.										(5)

b.	Explain the various methods used to correct irregular contours and oblique beam                  incidence.																      (10)

c.	How beam flatness and symmetry is determined?				            				(5)

6.	a.	Draw the typical electron beam PDD curve & define Rq, Rp, Rmax, R50land R90.    (10)

b.	Mention the various types of interaction of electrons with medium.					(4)

c.	Write a short note on electron arc therapy. 										(6)

(OR)

7.	a.	Explain in detail about the various dosimetric parameters of an electron beam.  	      (15)

b.	Discuss the various field shaping devices used to achieve a more customized electron

field shape.																	(5)

8.	Write an essay on stereotactic irradiation.

(OR)

9.	a.	Describe the various total body irradiation techniques with neat diagram.			(5)

b.	Mention the Physical and clinical requirements for total skin electron irradiation. 	(5)

c.	Write an essay on conformal radiotherapy.									      (10)