Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	12EC205						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention the code in which the consecutive numbers will have a single bit change.

2.	Mention the base of a hexadecimal number system.

3.	For a half adder circuit, what is the expression for CARRY?

4.	Give the equivalent representation of NOR gate using AND gate.

5.	Indicate the output of JK flip flop when both inputs are high.

6.	What is the use of excitation table?

7.	How many clock pulses are required to get the output of a 4 bit SIPO shift register?

8.	Mention the other name for ripple counter.

9.	Expand PAL.

10.	To implement full adder circuit in PROM, how many AND gates are required?

PART – B  (5 x 3 = 15 MARKS)

11.	Simplify the Boolean function

F (x,y,z) = ϵ (0,2,4,5,6)

12.	Design a Full adder circuit.

13.	Determine the characteristic equation of T Flip Flop.

14.	With neat diagram, show the difference between Johnson counter and ring counter.

15.	Implement F(A,B,C) = ∑ (3,4,6,7) using PLA.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Construct the truth table for the boolean functions

F1 = xyz'

F2 = x + y'z

F3 = x'y'z + x'yz + xy'

F4 = xy' + x'z

Draw the logic diagram for the same with minimum number of logic gates.			  (10)

b.	Simplify the following Boolean functions to a minimum number of literals:

a.   	xyz+x'y+xy {z'

b.   	zx+zx'y																    (5)

(OR)

17.	a. 	Represent the decimal number 8620

(i) in BCD 		(ii)	in excess-3 code 											    (4)

b.	Simplify the Boolean function by using Karnaugh map.

F(w, x, y, z) = Σ (0, 1, 2, 4, 5, 6, 8, 9, 12, 13, 14)									  (11)

18.	a.	Design a two bit magnitude comparator circuit and draw the logic diagram.			  (11)

b.	Design a Half Subtractor Circuit.												    (4)

(OR)

[P.T.O]

19.	a.	Design and implement a 4 bit Binary to gray code converter circuit.				    (9)

b.	Design a 4 to 1 Multiplexer Circuit.												    (6)

20.	a.	Draw the logic diagram of RS Flip Flop and obtain the characteristic table, characteristic 	equation, excitation table, truth table and graphic symbol.							  (12)

b.	Mention the difference between level triggering and edge triggering clock pulse.		    (3)

(OR)

21.	a.	With diagram, explain the need and operation of master slave JK Flip-flop.			    (8)

b.	Explain how to obtain D Flip Flop from SR Flip Flop using logic diagram. Mention the characteristic table, characteristic equation and excitation table of D Flip Flop.		    (7)

22.	a.	With neat diagram, explain the difference between Johnson counter and Ring counter. (7)

b.	Design a Binary asynchronous counter with its timing waveform.					    (8)

(OR)

23.	a.	Design a Mod 6 counter using T Flip Flop.										    (8)

b.	Explain the operation of SISO shift register with neat timing diagram.				    (7)

24.	a.	Explain the NAND operation with TTL logic in open collector output gate configuration. 																					    (8)

b.	With neat diagram explain the operation of TTL NAND gate with totem pole output.    (7)

(OR)

25.	Find minimum SoP for the following PoS and implement using

F(A,B,C,D)= π(2,4,6,7,11,12,14,15)

- PROM										       								    (5)

- PLA																			    (5)

- PAL											 								    (5)