Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	MICROPROCESSOR AND MICROCONTROLLER	      	     Time : 3 hours

Subject Code:	12EC246								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is size of address bus in 8085 microprocessor?

2.	How many flags are there in 8085?

3.	Give an example for three byte Instruction.

4.	___________ Interrupt of 8085 is Non Maskable.

5.	Write any two handshake signals.

6.	Write the BSR control word format in 8255.

7.	8051 architecture consists of Internal RAM of ___________ bytes.

8.	DPTR stands for ___________.

9.	What is the need for a Watchdog Timer?

10.	___________ type of A to D converter is commonly used.

PART – B  (5 x 3 = 15 MARKS)

11.	Specify the Number of T-states for the following instructions:

a) MVI A, 03H			b) LDA 6000H.

12.	What are the various types of machine cycles?

13.	Discuss the features of 8279 Programmable Keyboard/Display Interface.

14.	Name the addressing modes of 8051.

15.	What is the usage of TIMER and COUNTER?

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat sketch draw the block diagram of the architecture of 8085 Microprocessor. Explain the individual blocks in detail.

(OR)

17.	a. 	What are Addressing Modes? Explain the different types of addressing modes with an 			example.																	    (8)

b. 	Write an assembly language program to find the given number is odd or even.		    (7)

18.	With timing diagram, explain the memory read operation and memory write operation in 8085 microprocessor.

(OR)

19.	a. 	Define Instruction cycle, Machine cycle, T-state.     				            			    (7)

b. 	Draw and explain the timing diagram for the instruction 4100: STA 5000.         		    (8)

20.	With a block diagram explain each block of 8255 Programmable Peripheral Interface in detail.

(OR)

21.	Draw the block diagram of 8259 Programmable Interrupt Controller and explain the function of each block.

[P.T.O]

22.	Draw the architecture of 8051 microcontroller and explain the function of each block.

(OR)

23.	a.	Write an assembly language program to count the number of ones in a byte. If it is an

odd number store the result as 00 or else FF using 8051. 					  		(8)

b.	Write an ALP to find the factorial of a given number using 8051.			  		(7)

24.	Discuss in detail timer/counter mode operations in 8051.

(OR)

25.	Interface a stepper motor with 8051 and explain its operation with necessary diagrams.