Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	LOGIC AND DISTRIBUTED CONTROL SYSTEMS      	     Time : 3 hours

Subject Code:	12EI225								 			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the basic components of Data Loggers?

2.	SCADA stands for __________.

3.	What is the use of latching?

4.	Sketch a typical counter instruction.

5.	The different data comparison instructions available in PLC are __________.

6.    Mention any two arithmetic functions used in PLC.

7.    What does the term Data Highway refer to in DCS?

8.    Write any two Local Control Unit languages.

9.	List out three Industrial Communication Standards.

10.	Mention any two process industries where DCS is used for control applications.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between Data loggers and Data Acquisition System.

12.	Design an alarm system for the following condition. There are four hazards inputs to the alarm system.

a.	If one input is ON nothing happens.

b.	If any two inputs are ON a red pilot light goes ON.

c.	If any three inputs are ON an alarm siren sounds.

d.	If all four are ON the fire department is notified.

13.  Write a short note on data move operation in PLC.

14.	Explain the function of an Operator Station Unit of a DCS.

15.	List the different topologies used in DCS.

PART – C  (5 x 15 = 75 MARKS)

16.  Explain various digital controller Algorithms used in Direct Digital Control.

(OR)

17.	Draw the block diagram of SCADA system and explain each block.  Also explain the functions performed by SCADA systems.

18.	Explain different types of timer with suitable examples.

(OR)

19.	Write a PLC Program to obtain the following task

[P.T.O.]

- The conveyer motor is ON when the start push button is pressed

- Motor ON condition is indicated by pilot light Run

- Whenever the package senses the limit switch LS the motor stops.

- Motor OFF condition is indicated by Stop

- When emergency stop push button is pressed the motor stops at wherever the package is.

20.	Explain the following topics:

a. 	PID Function in PLC.															    (8)

b. 	Skip and MCR Function.														    (7)

(OR)

21.	Explain in detail about the sequencer functions in PLC with necessary example.

22.	Draw the block diagram of DCS and explain the building blocks in detail.

(OR)

23.	Explain the concept of redundancy implemented in DCS.

24.	Explain the various types of operator displays with a neat diagram.

(OR)

25.	Explain High level Engineering Interfaces in DCS.