Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ay – 2016

Subject Title :	PRESTRESSED CONCRETE STRUCTURES		      	     Time : 3 hours

Subject Code:	14CE202								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the advantage of using prestressed concrete in construction.

2.	State the range of grade of concrete for prestressed concrete members.

3.	Confirm whether tensile stresses are permitted in type I structures as per IS code.

4.	Draw the profile of parabolic cable.

5.	List out the factors affecting the deflection of prestressed concrete.

6.	Why do we provide reinforcement in anchorage zone?

7.	What do you mean by Concordant cable?

8.	Write down the other name for centroidal line of steel in prestressed concrete.

9.	Which IS code is to be used for the design of prestressed concrete pipes?

10.	Give some examples for prestressed tension and compression members.

PART – B  (5 x 3 = 15 MARKS)

11.	List the different types of losses of prestress.

12.	Draw the different types of tendon profile used in prestressed concrete beams.

13.	Sketch the arrangement of anchorage zone reinforcement.

14.	Distinguish between composite prestressed concrete beam with concrete beam.

15.	Write the main advantages of pre-stressed concrete pole.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the different losses of pre-stressed concrete.

(OR)

17.	A pre-tensioned concrete beam of 100mm wide and 300mm deep is prestressed by straight wires carrying an initial force of 150kN at an eccentricity of 50mm. The modulus of elasticity of steel and concrete are 210 and 35kN/mm2 respectively. Estimate the percentage loss of stress in steel due to elastic deformation of concrete if the area of steel wire is 188mm2.

18.	A post tensioned bonded prestressed concrete beam of rectangular section is 200mm wide and 400mm deep. The beam is prestressed by 300mm2 of high tensile steel located at an eccentricity of 100mm. The effective prestress after all losses is 800N/mm2. The characteristic tensile strength of steel is 1600N/mm2. Grade of concrete is M40. Estimate the ultimate moment capacity of the section.

(OR)

19.	Calculate the limit state of moment of resistance at the mid-span section of a slab for the following data. Depth of the section – 440mm, Area of prestress– 2770mm2, Grade of concrete – M45, Tensile strength of steel – 1520N/mm2,  Prestressing force – 3050kN, Young’s modulus of steel – 2.1x105N/mm2.

[P.T.O]

20.	A rectangular beam of c/s 350 x 850mm is subjected to an effective prestressing force of 2000 kN acting at the centroid of the section. fci=fck=35 N/mm2, fy=415 N/mm2 for ties and loss of prestress 20%, the cables pass through a steel plate symmetrically in an area of 200 x 300mm. Design the anchor plate and the reinforcements for bursting and spalling tensile forces by IS Method.

(OR)

21.	The end block of a post tensioned pre-stressed member is 550mm wide and 550mm deep. Four cables each made up of 7 wires of 12mm diameter strands and carrying a force of 1000kN are anchored by plate anchorages, 150mmx150mm located with their centers at 12mm from the edges of the end block. The cable duct is 50mm diameter, fck-45N/mm2 and fci - 25N/mm2. Yield stress of steel anchorage reinforcement is 260N/mm2. Design suitable anchorages for end block.

22.	Write short notes on the following:

a.	Guyon’s method.																    (7)

b.	Magnel’s method.															    (8)

(OR)

23.	Two simply supported beams, AB = BC = 10m of rectangular cross section, each post tensioned by means of two parabolic cap cables(p=300kN) with eccentricities of zero at supports and 150mm @ mid span. The ends of cap cable are located at 3m from the central support. The cable centre is 50mm from the top of the beam over the central support B. The beam is 200mm wide and 600mm deep.

a.	Calculate the secondary moment at B.

b.	Locate the resultant line of thrust through the beam AB.

c.	Evaluate the resultant pre-stress.

24. 	a. 	Discuss the design consideration of pre-stressed concrete sleepers.					    (8)

b. 	Explain the criteria of designing pre-stressed concrete pipes.						    (7)

(OR)

25. 	Write short notes on the following:

a. 	Advantages of Prestressed concrete poles.										    (5)

b. 	Design considerations of Prestressed concrete poles.								    (5)

c. 	Design criteria for Prestressed concrete pipes.                                        				    (5)