Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	INTERNET ROUTING ARCHITECTURE			      	     Time : 3 hours

Subject Code:	14CS2027								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Points of presence?

2.	Distinguishing the provider's network, equipment, and responsibilities from those of the customer is called as __________.

3.	What is meant by dynamic routing?

4.	List the different types of BGP message.

5.	Name the three routing information bases (RIBs) used in BGP routing table.

6.	The traffic that leaves the AS from a certain exit point returns through the same point then it is called as __________.

7.	What is the benefit of having confederations in a network?

8.	List the ways by which Non-BGP routers inside the autonomous systems can reach the outside world.

9.	__________ command is used in router configuration to display the IP routing table.

10.	What is Route dampening?

PART – B  (5 x 3 = 15 MARKS)

11.	What criteria’s should a customer use while selecting an ISP backbone?

12.	Differentiate distance vector and link state routing protocols.

13.	How do backdoor routes affect the traffic flow when different IGPs and EGPs are working together?

14.	Write short notes on the factors that affect route instabilities on the Internet.

15.	Write the configuration to control redistribution by creating a prefix list called FILTER with a first sequence number of 5 that explicitly permits the 172.16.1.0/24 network. Add a second line to the FILTER prefix list that explicitly permits all prefixes in the 10.0.0.0/8 address space that have a netmask between 16 and 24 bits and denies all other traffic.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the origin and the evolution of Internet.									    (7)

b.	Explain the major tasks of the four Router Arbiter Projects for routing administration.    (8)

(OR)

17.	a.	Elaborate on the different services offered by ISP and technical characteristics in detail. 																					  (10)

b.	SmallCo Technologies has been assigned IP network 215.43.10.0/24. Headquarters, with 49 hosts, is connected in a hub-and-spoke configuration with WAN links to four remote offices each with 20 hosts. Determine an appropriate IP addressing scheme to accommodate all the networks.													    (5)

18.	a.	Discuss the ways by which the segregating the worlds in Autonomous system results in a 	more manageable networks.													  (10)

b.	Describe the steps to perform basic routing for exchanging traffic between end stations. (5)

(OR)

[P.T.O]

19.	Explain the working of BGP with its message header format and different message types.

20.	What are the four categories of BGP path attributes? Discuss each of these attributes that are part of BGP UPDATE message for controlling BGP routes.

(OR)

21.	Discuss designing redundancy, symmetry and load balancing for single-homing and multi-homing scenario.

22.	a.	Explain how policy routing is followed for controlling the flow of traffic.			  (10)

b.	How to avoid routing loops when BGP policies conflict with internal defaults?		    (5)

(OR)

23.	a.	Illustrate the ways by which large scale autonomous system can be managed. 		  (10)

b.	Discuss the BGP tools for building core stability on the Internet.					    (5)

24.	a.	Demonstrate how to configure the BGP peering sessions for the below given topology. Use 	OSPF as an IGP to establish the required underlying connectivity internally.

(10)

b.	Explain the BGP 4 Aggregation with example.									    (5)

(OR)

25.	a.	Brief on the implementation and configuration of dynamically and statically default routes 		with an example scenario.														    (7)

b.	Consider a scenario where a customer has multiple connections to the Internet via the same 	provider. Provide the configuration of the routers for the following cases with example.

i.	Default only, one primary link and one backup link

ii.	Default primary, and backup, plus partial routing.							    (8)