# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

# Subject Title:	COMBUSTION IN ENGINES 						       Time : 3 hours

# Subject Code:	14ME3001								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the First law and second law application for combustion process.		      (10)

b.	Liquid octane (C8H18) enters the combustion chamber of a gas turbine steadily at 1 atm 		and 25°C, and its burned with air that enters the combustion chamber at the same state as 		shown in figure. Determine the adiabatic flame temperature for (a) complete combustion 		with 100% theoretical air, (b) complete combustion with 400% theoretical air and

(c) incomplete combustion (some CO in the products) with 90% theoretical air.	      (10)

Combustion

Chamber

Air 25C 1 atm

C8H1825C 1 atm

1 atm

CO2, H2O, N2, O2

2.	Explain the theories of combustion homogenous and heterogeneous mixtures.

(OR)

3.	a.	Explain the various pollutants generated during the combustion of fuel in I.C engines.

(10)

b.	List the various methods of monitoring the I.C engine exhaust pollutants.		      (10)

4.	Describe the various stages of combustion flame front propagation.

(OR)

5.	a.	Explain the methods of Charge Stratification and Combustion in SI engine.		      (10)

b.	The following observations were made during a trail on a single cylinder double acting 		condensing type steam engine (running at part load). Draw the heat balance sheet.     (10)

Pressure of steam supply						=    8bar

Quantity of condensate available from condenser	=   95  kg⁄h

Quantity of steam supplied to the engine		=   dry and saturated

Vacuum in condenser							=   300 mm of mercury

Engine speed									=    150 r.p.m

m.e.p on cover end side						=   1.52 bar

m.e.p on crank end side						=   1.22 bar

Diameter of the cylinder						=   200mm

Length of the stroke							=   300mm

Piston rod diameter							=   38mm

Net load on the brake							=   950 N

Effective diameter of brake wheel				=   0.75 m

Quantity of cooling water used					=   45 kg/min

Temperature rise of cooling water				=   18℃

Condensate temperature						=   55℃

6.	a.	Explain the various stages of combustion in a CI engine.						      (10)

b.	Define ignition delay. List the factors affecting ignition delay.					      (10)

[P.T.O]

(OR)

7.	a.	With a neat sketch, describe the working of air swirl flow device.				      (10)

b.	Explain with a neat sketch, any one design of CI engine combustion chamber.	      (10)

8.	Explain in brief, the requirements of a gas turbine combustion chamber.

(OR)

9.	Write short notes on:

a.	Flame stabilization and													      (10)

b.	Fuels for gas turbine.														      (10)