Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FUNDAMENTALS OF WIRELESS COMMUNICATION

Time : 3 hours

Subject Code:	14EC2044								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How do you increase the range of operation of a Bluetooth radio?

2.	Explain the concept of frequency reuse.

3.	Define reuse distance.

4.	What is co-channel interference?

5.	Compare TDMA, FDMA and CDMA.

6.	When is a handoff initiated?

7.	Explain cell dragging.

8.	How does cell splitting increase the capacity of a cellular system?

9.	What is meant by Doppler shift?

10.	Explain intersymbol interference.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the challenges in wireless networking?

12.	Briefly describe the umbrella cell approach.

13.  Derive the expression for simplified path loss model.

14.  Express the average delay spread, RMS delay spread in terms of power delay profile.

15.  Compare and contrast narrowband and wideband fading models.

PART – C  (5 x 15 = 75 MARKS)

16.	Write short notes on

a.	Satellite networks                                                                       						    (7)

b.	Cordless phones.				                                      							    (8)

(OR)

17.	Describe the evolution of cellular systems.

18.	Elaborate on the techniques to improve the capacity and coverage of a cellular system.

(OR)

19.	a.	If a total of 33 MHz of bandwidth is allocated to a particular FDD cellular telephone 	system which uses two 25 kHz simplex channels to provide full duplex voice and control 	channels, compute the number of channels available per cell if a system uses

(i) 4-cell reuse, (ii) 7-cell reuse (iii) 12-cell reuse. 								    (5)

b.	If a signal to interference ratio of 15 dB is required for satisfactory forward channel performance of a cellular system, what is the frequency reuse factor and cluster size that should be used for maximum capacity if the path loss exponent is  i) n= 4, ii) n=3? Assume that there are 6 co-channels cells in the first tier and all of them are at the same distance from the mobile. Use suitable approximations.									  (10)

[P.T.O]

20.	Discuss in detail the free space propagation model.

(OR)

21.	Derive the signal model that takes into account both the LOS path and the reflected path.

22.	Derive the time varying channel impulse response of statistical multipath channel model.

(OR)

23.	a.	Discuss in detail on the power delay profile, coherence bandwidth and the Doppler power 	spectrum of wideband fading models.                                                                		  (12)

b.	For a channel with Doppler spread BD = 80 Hz, find the time separation required in samples of the received signal in order for the samples to be approximated independent.                                                        																			    (3)

24.	a.	Explain the capacity offered by an AWGN channel in detail.  						  (12)

b.	What is Shannon capacity and outage capacity? 									    (3)

(OR)

25.	a.	Compare and contrast flat-fading and frequency-selective fading channels.       		    (3)

b.	Discuss on the capacity of time-invariant frequency-selective fading channels. 		  (12)