Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FUNDAMENTALS OF ELECTRONICS				      	     Time : 3 hours

Subject Code:	14EC2090										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	N type charges are called as _________.

2.	List out an application of UJT.

3.	Op amp is a linear device. (True/False)

4.	Draw the symbol of p channel JFET.

5.	State De morgan’s laws.

6.	When both the inputs are high, the output of NOR gate is__________.

7.	_________ is a volatile memory.

8.	_________ is more affected by noise.

9.	In _________ orbit the satellite rotates above the equator.

10.	What is the intermediate frequency in super heterodyne receiver?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the energy band diagram for a conductor.

12.	What are the ideal properties of op amp?

13.	Define the condition for Barkhausen criterion in an oscillator.

14.	Define semiconductor memory and list out its properties.

15.	Draw the basic block diagram of aerial system.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With neat sketch explain the characteristics, construction and working principle of PN 	diode. 																		  (10)

b.	Explain in detail the formation of intrinsic semiconductor and its characteristics.		    (5)

(OR)

17.	a.	Discuss the Common base configuration of BJT and plot the dc characteristics.		  (10)

b.	Explain the internal architecture of op amp with neat block diagram.				    (5)

18.	a.	List out the logic gates with the logic symbol, logic table and logic equation.		  (10)

b.	(875.12)10 = (  )8   																    (3)

c.	(1110101)2  = (  )16  															    (2)

(OR)

19.	a.	Explain the integrator circuit with a neat diagram. How will you overcome the problems of 	basic integrator circuit?														  (12)

b.	Derive the gain expression for an inverting op amp.								    (3)

20.	a.	What are the different types of memory? Discuss them in detail.					  (12)

b.	Brief virtual instrumentation.											    		    (3)

(OR)

21.	Explain the architecture of 8085 micro processor in detail with neat diagram.

[P.T.O]

22.	a.	Define frequency modulation. Derive the modulated signal of frequency modulation.   (10)

b.	What is the need for modulation? Explain in detail.								    (5)

(OR)

23.	a.	Explain the working of LVDT in detail with neat diagrams.						  (10)

b.	Briefly discuss about the telemetry systems.										    (5)

24.	a.	Explain the principle of super heterodyne receiver with neat block diagram.			  (10)

b.	List out the applications of MEMS and nano electronics.							    (5)

(OR)

25.	a.	What is noise? List the types of noise in detail.								  	  (10)

b.	Draw and explain the basic block diagram of communication system.				    (5)