# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

# Subject Title: 	SEISMIC DESIGN OF STEEL STRUCTURES		       Time : 3 hours

# Subject Code:	15CE3025 										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.		What are the factors that make the steel as the most ideal material for earthquake        	resistance?

b.	State and discuss the considerations for achieving adequate performance of steel buildings.

c.		Explain the Hysteretic behaviour of steel.

2.	a.	Explain the portal method of analysis of building frame subjected to horizontal forces 	with an example.

b.	Discuss the general features of the design of frames with eccentric bracings.

(OR)

3.	a.	Explain the cantilever method for analysing a building frame subjected to horizontal 	forces due to an Earthquake.

b.	How will you select the type of eccentric bracing?

c.	Write a note on strength and drift limitations for steel buildings.

4.	a.	Discuss about the evaluation of seismic design shear using lateral force method.

b.	Describe the concept of response spectra in dynamic analysis of building frames.

(OR)

X1

X2

X3

K1

K2

K3

M2

M3

M1

5.	a.	Consider a three storey shear building shown in figure with following properties.

m=300003000030 tonnes

Φ=1.001.001.000.548−1.522−6.2600.198−0.87212.10            ωn=3.889.15 rads

[P.T.O]

Compute the floor displacements, inter storey drifts, storey shears and over turning moments of this building when excited by an earthquake. The Pseudo spectral acceleration ordinates of the earthquake ground acceleration for the three modes are given as Sa = 2.94, 1.57 and 3.93m/s2. Assume the storey height to be 3m.

L1

L1

L1

L1

L1

L1

7.5m @ 2.5m each

12m @ 3m each

PLAN

6.	A four storey steel office building shown in figure is located in seismic zone IV on hard soil. The framing system of the building is moment resisting frames with brick masonry infill panels. Design the bracing system for the following data:

EQ

3.5m

3.5m

3.5m

3.5m

Sectional Elevation

Column section:  For ground floor ISHB 450 with 12mm thick cover plates on each flange and for remaining floors ISHB 450

Beam section: ISMB 400 for 7.5mm intermediate beams and ISMB 225 for all other beams.

Slab: 120mm thick RCC slab on all floors

Walls: 230mm thick having unit weight 18kN/m3

Bracing: Concentric

(OR)

7.	a.	Discuss about the design principles for special moment resistant frames used for steel 	buildings.

b.	Write a note on seismic design of frames with concentric bracing and dissipative 	connection.

8.	a.	Discuss about the analysis and design criteria for frames with X and V bracings and 			dissipative connections for the diagonals.

b.	Explain how RC structures are improved by incorporating steel concrete composite 			columns.

(OR)

9.	a.	What types of connections would you recommend for moment resisting frames in 			seismic areas and why?

b.	Discuss about the various techniques adopted for strengthening of steel buildings.