# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

# Subject Title: 	MATHEMATICAL THEORY OF ELASTICITY		       Time : 3 hours

# Subject Code: 	16MA3001										  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Derive the Cauchy’s stress formula.											      (10)

b.	The following displacement field is imposed on a body  u = (xyi + 3x2zj +4k)10-2

Consider a point P and a neighbouring point Q where PQ has the following direction 	cosine  nx = 0.200, ny = 0.800, nz = 0.555.  Point P has coordinates (2, 1, 3).  If PQ = Δs, 	find the components of P’Q’.												      (10)

2.	a.	Derive the Winkler-Bach formula for bending of curved beams.				      (10)

b.	Determine the maximum tensile and maximum compressive stresses across the section AA of the member loaded, as shown in Figure below.  Load P = 19620 N.		      (10)

(OR)

3. 	Derive the expression for maximum shear stress and displacement for a circular and elliptical bar subjected to torsion.

4. 	a.	A steel shaft of 10 cm diameter is shrunk inside a bronze cylinder of 25 cm outer 	diameter.  The shrunk allowance is 1 part per 1000 (i.e., 0.005 cm difference between the 	radii).  Find the tangential stress in the bronze cylinder at the inside and outer radii and 	the stress in the shaft.  Adopt Esteel = 214 x 106 kPa, Ebronze = 107 x 106 kPa and υ = 0.3 	for both metals.															      (10)

b.	Derive the expression for stresses due to gravitation.							      (10)

(OR)

[P.T.O]

5. 	a.	A flat steel disk of 75 cm outside diameter with a 15 cm diameter hole is shrunk around a 	solid steel shaft.  The shrink-fit allowance is 1 part in 1000 (i.e., an allowance of 0.0075 	cm in radius).  E = 214 x 106 kPa.

(i) What are the stresses due to shrink-fit?

(ii) At what rpm will the shrink-fit lossen up as a result of rotation?

(iii) What is the circumferential stress in the disk when spinning at the above speed?

Assume that the same equations as for the disk are applicable to the solid rotating shaft also.																	      (15)

b.	A solid steel propeller shaft, 60 cm in diameter, is rotating at a speed of 300 rpm.  If the shaft is constrained at its ends so that it cannot expand or contract longitudinally, calculate the total longitudinal thrust over a cross section due to rotational stresses.  Poisson’s ratio = 0.3.  The weight of steel may be taken as 0.077938 N/cm2.			(5)

6.	a.	Show that the resultant circumferential force across any radial section of a hollow disk 	subjected to thermal loading is zero.										      (10)

b.	Determine the bending moment due to the circumferential stress across a section of a 	thin hollow disk subjected to radial thermal variation.							      (10)

(OR)

7.	Derive the expression for σz and radial displacement ur for long circular cylinder and solid cylinder of radius ‘b’ and hollow cylinder with inner radius ‘a’.

8.	Obtain the expression for various elastic constants for the following cases:

a.	Transversely isotropic														      (10)

b.	Orthotropic body															      (10)

(OR)

9.	a.	 Consider a graphite-epoxy laminate whose elastic constants along and perpendicular to 	the fibres are as follows.  Exx = 181 GPa, Eyy = 10.3 GPa, Gxy  = 7.17 GPa, υyx = 0.28 and 	υxy = 0.01594.  Obtain the compliance coefficients appropriate to x’y’ axes which are at  	(a) +30 degree counter clockwise to xy axes and (b) +90 degree to xy axes.	 	      (10)

b.	A unidirectionally reinforced composite of  ‘Toray’ filament and ‘Nameo’ resin  has the 	following modulii and Poisson’s ratio.  Exx = 181 GPa, Eyy = 10.3 GPa, Gxy  = 7.17 GPa  	and υxy = 0.0159.  (1-VxyVyx)-1 = 1.0045.  Estimate the components of modulii for an off-	axis orientation of  (a) θ = +30 degree and (b) θ = +45 degree.					      (10)