Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	Reinforced Concrete Structures - I      	     Time : 3 hours

Subject Code:	09CE210/ 12CE217/ CE249		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is reinforced concrete?

2.	Define: Modular Ratio.

3.	What is a characteristic load?

4.	Write any one primary function of stirrups.

5.	What is a lintel beam?

6.	Why it is necessary to combine the footings? Give one reason.

7.	Define Short Column.

8.	Define the term flexure.

9.	Which was the traditional method of design not only for reinforced concrete, but also for

structural steel and timber?

10.	Write any one function of distribution bars in slabs.

PART – B  (5 x 3 = 15 MARKS)

11. Why the over-reinforced design is not preferred?

12.  What are the different types of shear reinforcement?

13. What is the main difference in terms of structural behavior between a short column and long  column?

14.  What are the three conditions for a beam section to behave as a flanged beam?

15.  Distinguish between one way slab and two way slab.

PART – C  (5 x 15 = 75 MARKS)

16.	A reinforced rectangular concrete beam 230 mm wide and 400 mm effective depth is subjected

to a moment of 42 kNm. The effective cover of compressive reinforcement is 40mm. Find the

reinforcing steel. Assume M20 concrete and Fe415 steel. Use WSD.

(OR)

17.	a.	Draw and explain the idealized stress strain graph for steel and concrete.			    (7)

b.	Explain the principle of Limit State design.										    (8)

18.	A hall has a clear dimension 3m x 9m with wall thickness 230mm. the live load on the slab is

3kN/m2 and a finishing load of 1kN/m2 may be assumed. Using M20 grade concrete and Fe

415 grade steel, design the slab.

(OR)

19.  A room of a residential building measures 4.3m x 6.55m. It is supported on 350mm thick walls on all four sides. The slab is simply supported at edges with no provision to resist torsion at corners. Design the slab using M20 concrete and Fe415 steel.

20.  Design the reinforcement in a column of size 450 mm x 600 mm subject to an axial load of 2000 kN under service dead and live loads. The column has an unsupported length of 3m and is braced against sidesway in both directions. Use M20 concrete and Fe415 steel.

[P.T.O]

(OR)

21. Design a plain concrete footing for a column 300 mm x 300 mm carrying an axial  load of 330 kN (under service loads, due to dead and live loads). Assume an allowable soil bearing pressure of 360 kN/m2 at a depth of 1m below ground. Assume M20 concrete and Fe415 steel.

22. 	A simply supported rectangular beam of 4m span carries a UDL of 26 kN/m. The width of the

beam is 230mm. Find the depth and steel area for balanced design. Use M20 concrete and Fe415 steel. Use Working Stress Method.

(OR)

23.	Write down the design procedure for T – beam in working stress method.

24.	Write down the design procedure for a one way slab in working stress method.

(OR)

25.	Design a column 10m long to carry an axial load of 600 kN. The column is restrained at the ends. Use M25 concrete.