Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	SOLID STATE CIRCUITS-I							      	     Time : 3 hours

Subject Code:	EC205						 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Peak Inverse Voltage for a diode.

2.	Mention the factors affecting the output voltage of a dc power supply.

3.	What is the stability factor of fixed bias BJT circuit?

4.	Write the Shockley’s equation of drain current in JFET.

5.	What are the advantages of RC coupled amplifier?

6.	What is the efficiency of series fed class A power amplifier?

7.	Mention the types of negative feedback connection.

8.	Define CMRR in differential amplifier.

9.	What do you mean by sustained oscillation?

10.	Give the relationship between the quality factor and bandwidth of a single tuned amplifier system.

PART – B  (5 x 3 = 15 MARKS)

11.	Derive the rectifier efficiency of Half Wave Rectifier.

12.	Sketch the variation of Q-point with respect to changes in IB, Rc, and Vcc for fixed bias BJT circuit.

13.	Define crossover distortion. How it can be eliminated?

14.	Derive the voltage gain for current series feedback system.

15.	Mention the limitations of RC and LC oscillators.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the working principle of Full wave rectifier and derive its rectifier efficiency, ripple factor, and transformer utilization factor. Also discuss the merits and demerits of the circuit.

(OR)

17.	With neat diagram explain the operation of controlled transistor series regulator and derive the output voltage expression.

18.	Analyze the fixed bias network for BJT and give the procedure to fix the operating point. Also derive the stability factor for the network.

(OR)

19.	Explain the voltage divider biasing for FET and show the mathematical approach for calculating the drain current.

20.	What is the advantage of cascading many stages of amplifier together? With neat sketch explain the operation of RC coupled amplifier and discuss the application and limitations of the circuit.

(OR)

[P.T.O]

21.	Mention the different modes of operation of power amplifier. Derive the efficiency of Series fed and transformer coupled Class A power amplifier.

22.	For voltage shunt negative feedback connection, explain the following parameters: gain, input impedance, output impedance, noise, distortion and bandwidth.

(OR)

23.	How drift is eliminated in Differential amplifier? Draw the ac equivalent circuit and derive the gain for all the modes of operation.

24.	How sustained oscillations are generated by RC phase shift oscillator? Also discuss the merits, demerits and applications of the circuit.

(OR)

25.	a. 	Write short notes on crystal oscillator.											    (5)

b.	With neat sketch explain the operation of single tuned amplifier and draw the frequency response.																	  (10)