Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	SOLID STATE CIRCUITS - II						      	     Time : 3 hours

Subject Code:	EC206							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name any two non-sinusoidal waveforms.

2.	Give the condition under which the high pass RC circuit acts as a differentiator.

3.	No two active devices have exactly identical characteristics. (True/False).

4.	Which circuit is used to convert sine wave to square wave?

5.	What is the other name of Astable multivibrator?

6.	Mention any two applications of Monostable Multivibrator.

7.	Give the relationship between es, ed and et.

8.	Define flyback time.

9.	What are the types of Astable Blocking Oscillator?

10.	Mention the disadvantage of monostable blocking oscillator using base timing.

PART – B  (5 x 3 = 15 MARKS)

11.	Sketch the response of low pass RC circuit to step input and give the output voltage expression.

12.	Define transition time and commutating capacitor.

13.	Consider an astable multivibrator which is used to generate square wave using the following component values C1 =C2 =100 pF and R1 =R2 =10 kΩ. Calculate the pulse width, period and frequency of the output.

14.	Draw the transistorized miller sweep generator.

15.	Give the disadvantages of unidirectional diode gate.

PART – C  (5 x 15 = 75 MARKS)

16.	With neat sketch, explain the high pass RC network response to

a.	Step input		b. 	Pulse input		c. 	Exponential input

(OR)

17.	Define clipper and with neat sketch explain the operation of series and parallel clipper.

18.	What is the function of multivibrator? Explain the operation of fixed bias Bistable multivibrator and draw the waveforms associated with it.

(OR)

19.	Schmitt trigger circuit is used to convert sinusoidal waveform to square waveform. Justify and  also discuss the applications of the circuit.

20.	a. 	Explain the working principle of collector coupled Astable Multivibrator with neat 	waveforms.									            							  (10)

b. 	What do you mean by rounding distortion in Astable multivibrator and how it can be eliminated?																	    (5)

(OR)

21.	Explain the working principle of collector coupled monostable Multivibrator with neat waveforms and derive the pulse width expression.

[P.T.O.]

22.	For the exponential sweep circuit, define and derive the expressions for sweep speed error, displacement error and transmission error.

(OR)

23.	Explain the principle and operation of simple current time base generator and draw the output waveforms.

24.	Explain the operation of base timing Monostable blocking oscillator and neatly sketch its current waveforms. Discuss the pulse width and limitations.

(OR)

25.	Write short notes on

a.	Unidirectional diode gate														    (5)

b.	Bidirectional gate	using diode and transistor										  (10)