Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL COMMUNICATION						      	     Time : 3 hours

Subject Code:	EC249											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Nyquist rate.

2.	What is Quantization?

3.	What is coherent detection?

4.	Why MSK is termed as “shaped QPSK”?

5.	Give the power spectral density of noise.

6.	Give the block diagram of base band signal receiver.

7.	Define entropy.

8.	State Shannon’s Capacity theorem.

9.	Define processing gain.

10.	Enlist the properties of pseudo noise sequences.

PART – B  (5 x 3 = 15 MARKS)

11.	Derive the expression for Quantization noise.

12.	Prove that in DEPSK the noise always occurs in pairs.

13.	Write short notes on the non coherent detection of FSK.

14.	Draw and explain the working of (n, k) generic cyclic codes.

15.	Write short notes on anti jamming and multipath performance.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat block diagram explain the working of delta modulator.

(OR)

17.	What is ISI? Using suitable waveforms and diagrams comprehend on the ideal Nyquist channel.

18.	Using suitable waveforms and block diagram explain on QPSK transmitter, receiver and its working.

(OR)

19.	What is MSK? Using suitable block diagram and waveforms explain MSK.

20.	Derive the transfer function of Optimum filter.

(OR)

21.	Obtain the transfer function of Matched filter.

22.	In a (7,4) linear block code, construct the generator matrix, parity check matrix, error correcting capability, obtain all the code vector.

C5=d1+d2+d4;

C6=d3+d4;

C7=d1+d3+d4;

Decode the received sequence r(1101110)

[P.T.O]

(OR)

23.	Encode the message m(1111) using a (2,1,3) Convolutional encoder whose generator is defined as g1(111) and g2(101) using the maximum likelihood encoding. Draw also the encoder, state diagram and trellis of the encoder.

24.	Using a suitable block diagram explain the direct sequence spread spectrum.

(OR)

25.	Using suitable waveforms and block diagram explain the frequency hopping spread spectrum.