Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRON DEVICES								      	     Time : 3 hours

Subject Code:	EC283							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The number of electrons per coulomb is _________.

2.	_________ waveform voltage is used to sweep the electron beam across the screen in CRO.

3.	How semi conductors are classified?

4.	What is cut-in voltage?

5.	The diode current equation relating the voltage V and current I is given by _________.

6.	What are the types of transistor configuration?

7.	Draw the circuit symbol for a n-channel JFET.

8.	List the various terminals of SCR.

9.	Say True or False: LED consumes more power than the LCD.

10.	Say True or False: Life time of LCD is less than the LED.

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain insulator, semiconductor and conductor with the help of energy band structure.

12.	What is meant by Donor impurities?

13.	Compare CE, CC and CB configurations.

14.	Draw the equivalent circuit of an UJT and explain.

15.	State the applications of Tunnel diode.

PART – C  (5 x 15 = 75 MARKS)

16.	What is electrostatic deflection in a Cathode Ray Tube? Explain briefly.

(OR)

17.	Explain the magnetic deflection in a Cathode Ray Tube.

18.	Explain the operation of PN junction diode when it is forward biased and reverse biased. Discuss its Volt-Ampere characteristics.

(OR)

19.	Explain Hall effect. How can the properties of semi conductor are determined with the help of Hall Effect?

20.	a.	Explain the operation of NPN and PNP transistors.								    (8)

b.	Explain the input and output characteristics of a transistor in CB configuration.		    (7)

(OR)

21.	With neat diagram explain in detail about common emitter configuration and its input and output characteristics.

22.	Explain the construction and operation of a JFET with its static characteristics.

(OR)

23.	With neat diagrams, explain the construction and operation of SCR.

24.	a.	Describe, with relevant diagrams, the construction of a LED and explain its working.	  (10)

b.	Compare LED and LCD.														    (5)

(OR)

25.	Write short notes on:	a.	TRIAC		b.	Varactor diode						      (8+7)