Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	SIGNALS AND SYSTEMS							      	     Time : 3 hours

Subject Code:	EC285							 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Sketch the DT unit step signal.

2.	A Differential equation is used to represent a __________ system. (CT/DT)

3.	The quantity |X(jω)|  plotted against ω is termed as __________ of the signal.

4.	If F[jω] is Fourier transform of f(t) then Fourier transform of f(-t)= __________.

5.	What is meant by aliasing?

6.	The unilateral laplace transform is used to analyze __________ (causal/ non causal) system.

7.	Fourier transform, Laplace transform, and Z transform of the impulse signal is unity. (True/False)

8.	Find x(n) if x(e-jω)=1-e-jω+2e-2jω+2e-3jω

9.	An LTI system is stable if and only if the ROC of its system function H(Z) __________.

10.	ROC of a signal is defined as __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Determine whether the system described by the following differential equation is linear:

dy(t)/dt  +2y(t) = x(t)

12.	What are the necessary conditions for Fourier Transform?

13.	Find the Laplace transform of x(t) = e-b|t|

14.	Find the DTFT of the sequence x(n) = u(n).

15.	Find Z-transform of x(n)=(0.5)nu(-n)

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Check whether the system y(n) = x(n)x(n-1) is (a) Static or Dynamic (b) Linear or Non 	Linear (c)Causal or Non Causal (d)Time Invariant or Time Variant.				  (10)

b. 	Find the output of the system with input sequence x[n]=[1 2 3 4] and system having impulse response h[n]=[1 2 3].													    (5)

(OR)

17.	Sketch the following signal

a.	x[n] =u[-n+2] . u[n]

b.	x(t)=-2u(t-1)

c.	x(t)=3r(t-1)

d.	x(t)=r(t).u(2-t)

e.	x(t)= u(-7t+1)

18.	Consider a Stable LTI system characterized by the differential equation d2y(t)/dt2+4dy(t)/dt+3y(t)=2x(t)+dx(t)/dt

Find the impulse response using fourier transform and also determine the output for x(t)=e-t u(t)

(OR)

[P.T.O]

19.	Obtain the Cosine Fourier series expansion of a half wave rectified sine wave.

20.	How will you convert a CT signal to DT signal? Explain impulse train sampling method with neat diagrams.

(OR)

21.	a.	Find Laplace Transform for the following signal

x(t)=[e-tu(t)* cos(t-2) u(t-2)] 													    (5)

b.	A signal has laplace transform of X(s)=s+2/(s2+4s+5). Find the Laplace transform of following signal

y1(t)=tx(t);	y2(t)=x(t)*x(t)													  (10)

22.	State and prove any five properties of DTFT.

(OR)

23.	Consider a LTI system which is characterized by the difference equation

y(n)-(3/4)y(n-1)+(1/8)y(n-2)=2x(n)

Find the impulse response and determine the output of the system for the input x(n)=(1/4)n u(n).

24.	Perform long division method to find x(n) X(Z)=(1+2z-1)/(1-2z-1+z-2)							a.	x(n) is causal  																    (8)

b.	x(n) is anticausal												    				    (7)

(OR)

25.	a.	Using Z transform find the convolution of two sequences x1(n) = {1, 2, -1, 0, 3}

x2(n)={1, 2,-1}																    (5)

b.	Determine Z transform for the following signals

i.	x(n)=an-1u(n-1)

ii.	x(n) = 7(1/3)n u(n) - 6(1/2)n u(n) and plot its RoC.							  (10)